**

**

**  Decigel020	- The functionality of the famous "decigel" program, but

**  working on the 68020/68030 processors.

**

**  The old Decigel would correctly patch the instruction on the 68020,

**  but chances are the old (bad) instruction was still in the instruction

**  cache.  This code flushes the cache after modifying memory.

**

**  This code may not function under future revisions of the operating

**  system.  This code is safe on the 68000/68010/68020 and 68030.

**  This code is not expected to function on the 68040.  This code may

**  be called from the CLI only.

**

**

**  Written Tuesday 03-Apr-90 21:21:47 -Bryce Nesbitt

**

**

INCLUDE "exec/types.i"

INCLUDE "exec/memory.i"

INCLUDE "exec/ables.i"

INCLUDE "exec/execbase.i"

INCLUDE "libraries/dosextens.i"

INT_ABLES

XREF	_LVOFindTask

XREF	_LVOSupervisor

ABSEXECBASE	EQU 4

PrivTrapVector	EQU $20

;-------------- install patch then detach -----------------------------------

move.l	ABSEXECBASE,a6

;

;   Contents of the old vector are used to self-modify our

;   code.  The new vector replaces the old.

;

DISABLE

move.l	PrivTrapVector,ModifyCode+2

bsr.s	FlushCache

move.l	#NewPrivTrap,PrivTrapVector

ENABLE

;

;   Detach our code from the CLI

;

suba.l	a1,a1

jsr	_LVOFindTask(a6)

move.l	d0,a0

move.l	pr_CLI(a0),a0

add.l	a0,a0

add.l	a0,a0

move.l	a0,d0

beq.s	not_cli

clr.l	cli_Module(a0)

not_cli:	moveq	#0,d0

rts

*

*   Flush the instruction cache

*

FlushCache:	movem.l a5/a6,-(sp)

move.l	ABSEXECBASE,a6

btst.b	#AFB_68020,AttnFlags+1(a6)  ;>=68020 includes cache

beq.s	fc_nocache

lea.l	FlushTrap(pc),a5

jsr	_LVOSupervisor(a6)

fc_nocache:	movem.l (sp)+,a5/a6

rts

;

;

FlushTrap:	dc.w	$4e7a,$0002 ;movec.l CACR,d0

bset	#3,d0	    ;Set "Clear instruction cache" bit

dc.w	$4e7b,$0002 ;movec.l d0,CACR

rte

*****************************************************************************

**									   **

**									   **

**  The trap handler wedged into the privilege violation vector.	   **

**									   **

**  If the instruction was MOVE SR,<ea> it is converted to MOVE CCR,<ea>.  **

**  The instruction cache is flushed, then the instruction is re-executed. **

**									   **

**									   **

*****************************************************************************

STKOFFSET	EQU	4*3

;

;   New privilege violation vector

;

NewPrivTrap:	movem.l d0/a0/a6,-(sp)

move.l	STKOFFSET+2(sp),a0

move.w	(a0),d0             ; Examine opcode

andi.w	#~%111111,d0	    ; Mask out EA field

cmpi.w	#$40C0,d0	    ; A MOVE SR,<ea>?

beq.s	GotOne

movem.l (sp)+,d0/a0/a6

ModifyCode:	jmp	$01234567	    ; To previous handler... (exit)

;

;   Code executed if the instruction was MOVE SR,<ea>

;

GotOne: 	move.l	ABSEXECBASE,a6

DISABLE

bset	#1,(a0)             ; Convert to MOVE CCR,<ea>

btst.b	#AFB_68020,AttnFlags+1(a6)  ;>=68020 includes cache

beq.s	no_cache

dc.w	$4e7b,$8802 ; movec.l a0,CAAR

dc.w	$4e7a,$0002 ; movec.l CACR,d0

bset	#2,d0	    ; Set "Clear entry in instruction cache"

dc.w	$4e7b,$0002 ; movec.l d0,CACR

no_cache:	ENABLE

movem.l (sp)+,d0/a0/a6

rte			    ; Rerun new opcode... (exit)

END