TABLE OF CONTENTS

cdg.library/--background--

cdg.library/_CDGOpen

cdg.library/CDGBack

cdg.library/CDGBegin

cdg.library/CDGChannel

cdg.library/CDGDiskRemoved

cdg.library/CDGDraw

cdg.library/CDGEnd

cdg.library/CDGFastForward

cdg.library/CDGFront

cdg.library/CDGNextTrack

cdg.library/CDGPause

cdg.library/CDGPlay

cdg.library/CDGPrevTrack

cdg.library/CDGRewind

cdg.library/CDGStop

cdg.library/--background--                         cdg.library/--background--

CD+G

CD+G is a standard format for encoding graphics data on

audio CD's.  This data is stored with the audio data such

that the graphics data retrieved is synchronized with the

audio.

CD+G currently defines a limited set of graphics operations

including --

o Setting border color (0..15)

o Drawing a 6 bit wide, by 12 bit high graphic image

at a specific ROW/COLUMN position.  The graphic image

is drawn in 2 colors.  The pen colors to use are encoded

in the graphic data.

A 6x12 pixel element is known as a FONT.

o Exclusive-OR a 6x12 bit graphic image with another.

EOR makes it possible to draw a graphic image, and

color it in more than 2 colors by EORing the previous

bit-pattern with a new one using 2 new pen colors.

o Clear the entire display with a color (0..15).

o Change the color table (0..15).  Separate CD+G instructions

are used to load pen colors 0-7, and 8-15.

o Scroll screen left/right, up/down.  The vacated portion

is filled in with a pen color (0..15).  Smooth scrolling

is possible.

o Scroll screen left/right, up/down.  The vacated portion

is filled in with the data which is scrolled off screen.

There are currently two kinds of graphics --

o Line Graphics - the display is a VISIBLE 2 rows, by

48 columns.  Each FONT element in the display is 6 pixels

wide by 12 pixels high.  This mode is currently NOT

supported.

o TV Graphics - the display is a VISIBLE 16 rows, by

48 columns.  Each FONT element in the display is 6 pixels

wide by 12 pixels high.  This mode IS supported by

cdg.library.

CHANNELS -

Two of the CD+G drawing instructions can be divided into

channel numbers.  These are the WRITE FONT intruction

which draws a 2 color 6x12 image on the screen, and the EOR

FONT instruction which uses EOR logic to draw over an existing

font.

The primary channel number is 0, and would typically be used

to draw a picture.  Infact, some CD+G disks will only use

channel 0.

In addition to channel 0, there are 15 other channels which

can be used for additional graphics imagery, such as the

text for more than one language.

By default the cdg.library uses channels 0, and 1 for

graphic data.  The cdg.library also provides a function

so that channels 2-15 can be used instead of channel

1.  Therefore, two channels are always active.

While the CD+G specification recommends that there be

a mechanism for selectively turning on/off all 16 channels,

it is also not entirely clear if, or how the data should

be mixed on screen.

Because there are already variations in CD+G implementations

regarding channel selection, and usage, the scheme provided

for in cdg.library is probably the least complicated for the

user.  It is however admittedly not completely within the

ideal specification.

PACKS & PACKETS

CD+G data is stored in something called a PACKET.  There

are up to 4 TV GRAPHIC'S PACKS per PACKET.  Each PACK

in a PACKET can be a separate graphics instruction from

those listed above.

Because a new PACKET is available 75x/second, CD+G requires

most of the available CPU time on a 68000 based system to

keep up with the data.

ERROR CORRECTION

Because of the nature of CD's, data can be munged.  Because

audio data must be played in real-time, it is not possible

to retry a bad block of data.  The CD standard includes

an error CORRECTION method which helps to minimize data loss.

Error correction may, or may not be implemented in the

cdg.library.  Error correction requires a considerable

amount of CPU time, or error correcting hardware if

available.  Even with error correction it is not always

possible to correct the data.

The result of NON-CORRECTED data which is in error may be

a display glitch.

The result of NON-CORRECTABLE data which is in error is

missed graphics data, also resulting in a display glitch.

While better than no error correction, there is still the

possibility of an imperfect display.

cdg.library (V38)

The cdg.library provides a set of functions for use with

the Commodore CDTV player.  These functions are meant to be

used by the Player Preferences module which lets the user

select audio tracks to play.  Note that these functions

were not available in the initial cdg.library, and the V38

functions are not compatable with the earlier version of

the library.

The caller of cdg.library functions is responsible for managing

user input, and CD activity, including selecting tracks to

play, starting play, stopping play, etc.

In the typical case, the following steps are recommended -

1.) Initialize a CDGPrefs structure.  This structure

must be entirely filled in, and contains many pointers to

sprite data which must be initialized.

The sprites are used to display activity information to the

user when the CD+G display is active.  An example would be

a change in channel selection.  A new channel number is

automatically displayed for a few moments for the user's

benefit.  Sprites are also used to display disk activity

such as PAUSE mode.

Each image is composed of 4 sprites.  Two sprites are

used for the left side of the symbol, and two for the

right.  The image can use 8 colors because attached

sprites are used.  Each symbol requires pointers to

FOUR sprites arranged in this order -

1st ptr -> Left side sprite

2nd ptr -> Left side, attached sprite

3rd ptr -> Right side sprite

4th ptr -> Right side, attached sprite

The ordering arrangement is designed to be convienent

for the cdg.library, and provides enough colors to make

attractive images.  Because sprites are used, the total width

of each image is always 32 bits wide, and the height is

variable.  It is highly recommended that the images not be

more than 32 bits high to insure they fit on the screen, and

avoid excessive memory usage.

Sprite data should be in CHIP RAM, and be stored in the

simple sprite format including space for the posctrl words, and

NULL terminating words.  The height of all sprites must be

the same.

The CDGPrefs structure includes other fields defined in the

cdgprefs.i/h file such as display position, and an array of

8 colors to be used for the sprites.

2.) Call CDGBegin() with the CDGPrefs() structure as an argument.

This function starts the cdg.library.  The call may fail due to

lack of memory, or inability to open a needed system resource.

Of interest, the cdg.library will only process MIDI packets IF

the serial port, and control bits have not been allocated by

some other task (e.g., the serial.device).

Once started, the cdg.library starts collecting PACKETS as

available.  The PACKETS are buffered (though because of the

rate and size of the PACKETS), assume no more than 1 seconds

worth of PACKET data will be buffered before it is overwritten

by more recent PACKETS.

3.) Call CDGPlay(), and start the CD playing.  Once the CD starts

playing, assume the cdg.library is receiving CD+G PACKETS.  At this

point your code should call CDGDraw() as often as possible.  During

every call, CDGDraw() will process any pending PACKS.

CDGDraw() also returns a value indicating if ANY CD+G, or CD+MIDI

PACKS have been processed.  This is meant to be used as an indicator

that the CD+G screen should be brought to front, or perhaps a gadget

would be displayed indicating that CD+G is available.  The user will

probably prefer the CD+G screen automatically coming to front, or they

will miss the initial portion of the animation.

To bring the CD+G screen to front, use the CDGFront() call.

It is recommended that you first call CDGDraw() so that

any pending PACKS will be processed before the screen is

made visible.

4.) The player program is now responsible for switching to

CD+G mode by efficiently checking for user input, and busy

looping on the CDGDraw() function.  The loop should be coded

to provide maximum CPU time for CDGDraw().

5.) If the user presses a key, or button indicating they want

to bring the Player program to front, you must use the CDGBack()

function to tell CD+G to turn off any sprites it has on screen, and

notify it not to bring its own screen to front again until CDGFront()

is called.

6.) The cdg.library provides a number of functions for

telling it what the CD is doing, as well the ability to

change the active channels 1-15.  These are functions are also

used to provide some visual indication to the user that their input

has been noticed, and what is happening.  In addition to the channel

selection, there are sprite images in CDGPrefs for the following

modes --

PLAY	- Image is displayed for a few moments.

Indicates the disk has started PLAY.

Should be displayed whenever the disk

starts playing, including after PAUSE,

STOP, FF, REW, NEXT TRACK, PREVIOUS TRACK.

PAUSE	- Image is displayed indefinitely.

STOP	- Image is displayed indefinitely.

FF	- Image is displayed indefinitely.

REW	- Image is displayed indefinitely.

NEXT TRACK - Image is displayed for a few moments.

PLAY is implied.

PREV TRACK - Image is displayed for a few moments.

PLAY is implied.

CHANNEL    - Image is displayed for a few moments.

See AutoDocs for the functions used to display these images.

7.) If the CD is removed, the CDGDiskRemoved() function should

be called.

8.) To close down the cdg.library, use the CDGEnd() function.

Note that while your own screen is displayed, it is not

necessary to call CDGDraw() on a timely basis, however it

is important to call it often (e.g., once every 1/2-3/4

seconds) so that CD+G/+MIDI data is not lost.

Note that sprite images can be replaced by a more recent

sprite image if the image has not already been removed

from the display.  For example, if the user is changing

channel numbers, you can call CDGChannel() at any time,

and the most recent channel will be displayed.

If you call CDGBack() while a sprite image is being

displayed, the sprite image is turned off at the next

VBLANK.

SPECIAL CONSIDERATIONS

During FF and REW, CD+G data is missed.  Because data is

missed, the CD+G display is cleared during these operations,

and it can take a while for the display to be properly

updated.  For example, if the color tables were supposed to

have been changed during the section of the CD which is

skipped in FF/REW modes, the colors will be wrong when the

disk starts to PLAY again.

Because there is no work around for this problem, you can

either disable FF/REW during CD+G, or allow the problem.

Ideally CD+G instructions will contain enough repetative

information that this won't be a significant problem.  If

for example the CD+G data reloads the color tables often,

and redraws the screen often enough, the problem is tolerable.

MIDI DATA

CD's can also contain MIDI data.  The cdg.library optionally

handle MIDI PACKETS during CDGDraw(), and sends this data

at MIDI Baud rates out the serial port.  However because

of variations in MIDI implementations, the cdg.library

code may not work with all MIDI devices.  Incompatability

reports will be considered if a cdg.library upgrade is

implemented in the future.

cdg.library/_CDGOpen                                     cdg.library/_CDGOpen

NAME

CDGOpen -- Open library

SYNOPSIS

cdgbase = CDGOpen( CDGBase )

D0                    A6

struct CDGBase *CDGOpen( struct CDGBase * );

FUNCTION

Returns pointer to library base if the library has not been

previously opened.  Because of limited hardware resources

(e.g., sprites, and CPU bandwidth) only one CD+G display

is currently feasable.

Therefore this OpenLibrary() of "cdg.library" returns a

pointer to the library base only if some other task has not

already opened it.

You must call CDGBegin() with a CDGPrefs structure to actually

start CD+G.

RETURNS

Library base pointer, or NULL indicating the library is in use.

NOTES

cdg.library/CDGBack                                       cdg.library/CDGBack

NAME

CDGBack -- Called before you bring your own screen to front.

SYNOPSIS

VOID CDGBack( VOID )

VOID CDGBack( VOID );

FUNCTION

Tells CD+G you are going to bring your own screen in front

of CD+G's screen.  This tells CD+G that it should not display

any sprites, or bring its own screen to front.  The programmer

must call CDGBegin() first - failure to do so will result in a

crash.

RETURNS

N/A

NOTES

cdg.library/CDGBegin                                     cdg.library/CDGBegin

NAME

CDGBegin -- Start the CD+G display system

SYNOPSIS

result = CDGBegin( CDGPrefs )

D0                    A0

BOOL CDGBegin( struct CDGPrefs * );

FUNCTION

Starts CD+G with given preferences.  You must call CDGFront()

to bring the CD+G screen to front.  CD+G uses graphics.library

functions, and is not meant to be used in an Intuition

screen environment.

INPUTS

CDGPrefs -- pointer to a CDGPrefs structure.  The structure

includes the following information -

Sprite image data for CD+G Channel selection.

Sprite image data for CD play, pause, etc.

Top/Left edge corder of CD+G screen.

Interlaced display (TRUE or FALSE) - does not affect

display resolution.

RETURNS

TRUE if CD+G started.  FALSE indicates failure, most likely

due to lack of memory, or inability to obtain some other

resource.

NOTES

Once started, the CD+G library will start buffering CD+G PACKETS.

These packets will be lost if you do not call CDGDraw() often

enough.  The CD+G screen can be made visible by calling

CDGFront().

cdg.library/CDGChannel                                 cdg.library/CDGChannel

NAME

CDGChannel -- Set active channel number for FONT data

SYNOPSIS

VOID CDGChannel( channel )

D0

VOID CDGChannel( ULONG );

FUNCTION

Tells the cdg.library to use a new channel.  Channel 0 is always

displayed.  In addition, one of 15 other channels can also be

displayed.  The selected channel number is automatically displayed

on the CD+G screen for a reasonable amount of time (approximately

2-3 seconds).

INPUTS

channel - A number from 1-15.  Error checking is not performed; you

must pass in valid values.

RETURNS

N/A

NOTES

The caller is responsible for managing user input, including

polling for some button press indicating that the user would

like to increment, or set a new active channel.

cdg.library/CDGDiskRemoved                         cdg.library/CDGDiskRemoved

NAME

CDGDiskRemoved -- Tell CD+G that the CD has been removed

SYNOPSIS

VOID CDGDiskRemoved( VOID )

VOID CDGDiskRemoved( VOID );

FUNCTION

Tell the cdg.library  that the CD has been removed.  The CDTV "+G",

and/or "+MIDI" control panel lights are turned off (if they are

already on).

INPUTS

N/A

RETURNS

N/A

NOTES

The the cdg.library turns on the "+G", and/or "+MIDI" front panel

lights if a +G, or +MIDI PACK has been processed during

CDGDraw().  There is currently no good way to determine if

a disk is +G, or +MIDI before a track has been played.

cdg.library/CDGDraw                                       cdg.library/CDGDraw

NAME

CDGDraw -- Main CD+G PACKET handling routine

SYNOPSIS

flags=CDGDraw( VOID );

D0

UWORD CDGDraw( VOID );

FUNCTION

Process pending packets.  CDGDraw() must be called in

a loop from an efficient, relatively high priority task.

Failure to call this function often enough will result

in CD+G data being lost.

Currently this function must be called at least once every

3/4 of a second, but while the CD+G screen is in front, it

should be called as frequently as possible.  Ideally it is

called in a loop which does little more than check for

user input, and use the cd.device to control disk play,

stop, pause, etc.

RETURNS

flags - A set of flags as defined in cdgprefs.h/i.  The

flags are used by the caller to determine if any +G, or

+MIDI PACKS have been processed.

NOTES

Currently this function knows how to process TVGRAPHICS

PACKS, and MIDI PACKS.  MIDI PACKS causes data to be sent

at MIDI baud rates via the serial port.  The first MIDI

PACK received also causes MIDI initialization information

to be transmitted.

It is also possible to use Wait() instead of calling this

function in a pure busy loop.  This requires providing a

pointer to a task structure, and signal mask in the CDGPrefs

structure when you call CDGBegin().  Because your task

will be signaled approximately 75 times per second, your

code needs to be efficient.

In addition, the calling task must be of a high enough

priority to avoid long delays.  In the typical CDTV application

only one task is effectively running, so this may not be

a significant problem.  Even short delays can result in

rough animation which is undesirable.

It is allowable to call this function a pure busy loop,

and not wait for a signal.  This function will simply

return if there are no PACKs to be processed.

cdg.library/CDGEnd                                         cdg.library/CDGEnd

NAME

CDGEnd -- End CDG playing

SYNOPSIS

VOID CDGEnd( VOID )

VOID CDGEnd( VOID );

FUNCTION

Stops CD+G.  All memory is freed.  Should never be called

if CDGBegin() failed.

RETURNS

N/A

NOTES

cdg.library/CDGFastForward                         cdg.library/CDGFastForward

NAME

CDGFastForward -- Tell CD+G that the disk is in FF mode

SYNOPSIS

VOID CDGFastForward( VOID )

VOID CDGFastForward( VOID );

FUNCTION

Tell the cdg.library that the disk is in the FF mode.

Sprites, passed in a CDGPrefs structure at CDGBegin()

time, are used to display the mode.

INPUTS

N/A

RETURNS

N/A

NOTES

The FF mode sprite(s) is displayed until CDGPlay() is called,

CDGBack() is called, or some other cdg.library mode function

is called.

cdg.library/CDGFront                                     cdg.library/CDGFront

NAME

CDGFront -- Bring CDG screen to front

SYNOPSIS

VOID CDGFront( VOID )

VOID CDGFront( VOID );

FUNCTION

Brings CD+G screen to front.  Programmer must call CDGBegin()

first - failure to do so will result in a crash.

RETURNS

N/A

NOTES

Recommend calling CDGDraw() once before calling this function.

This will cause the CD+G display to get up to date before

its screen is brought to front.  Do not call this function

in a busy loop; not needed.

See CDGBack() which should be called before bringing your

own screen to front.

cdg.library/CDGNextTrack                             cdg.library/CDGNextTrack

NAME

CDGNextTrack -- Tell CD+G that the disk is in NEXT TRACK mode

SYNOPSIS

VOID CDGNextTrack( VOID )

VOID CDGNextTrack( VOID );

FUNCTION

Tell the cdg.library that the disk is in the NEXT TRACK mode.

Sprites, passed in a CDGPrefs structure at CDGBegin()

time, are used to display the mode.

INPUTS

N/A

RETURNS

N/A

NOTES

The NEXT TRACK mode sprite(s) is automatically turned off after

a few moments (e.g., 1 second), or when CDGBack() is called.

cdg.library/CDGPause                                     cdg.library/CDGPause

NAME

CDGPause -- Tell CD+G that the disk is in PAUSE mode

SYNOPSIS

VOID CDGPause( VOID )

VOID CDGPause( VOID );

FUNCTION

Tell the cdg.library that the disk is in PAUSE mode.

Sprites, passed in a CDGPrefs structure at CDGBegin()

time, are used to display the mode.

INPUTS

N/A

RETURNS

N/A

NOTES

The PAUSE mode sprite(s) is displayed until CDGPlay() is called,

CDGBack() is called, or some other cdg.library mode function

is called.

cdg.library/CDGPlay                                       cdg.library/CDGPlay

NAME

CDGPlay -- Tell CD+G that the disk is in PLAY mode

SYNOPSIS

VOID CDGPlay( Show )

D0

VOID CDGPlay( BOOL );

FUNCTION

Tell the cdg.library that the disk is in PLAY mode.  You must call

this function each time you start the CD playing to enable

subcodes (R through W).

Sprites, passed in a CDGPrefs structure at CDGBegin()

time, are used to display the mode.

INPUTS

Show -- If TRUE, the PLAY sprite is displayed on the

CD+G screen for a few moments.

RETURNS

N/A

NOTES

The PLAY mode sprite(s) is automatically turned off after

a few moments (e.g., 1 second), or when CDGBack() is called.

cdg.library/CDGPrevTrack                             cdg.library/CDGPrevTrack

NAME

CDGPrevTrack -- Tell CD+G that the disk is in PREV TRACK mode

SYNOPSIS

VOID CDGPrevTrack( VOID )

VOID CDGPrevTrack( VOID );

FUNCTION

Tell the cdg.library that the disk is in the PREV TRACK mode.

Sprites, passed in a CDGPrefs structure at CDGBegin()

time, are used to display the mode.

INPUTS

N/A

RETURNS

N/A

NOTES

The PREV TRACK mode sprite(s) is automatically turned off after

a few moments (e.g., 1 second), or when CDGBack() is called.

cdg.library/CDGRewind                                   cdg.library/CDGRewind

NAME

CDGRewind -- Tell CD+G that the disk is in REW mode

SYNOPSIS

VOID CDGRewind( VOID )

VOID CDGRewind( VOID );

FUNCTION

Tell CD+G that the disk is in the REW mode.

Sprites, passed in a CDGPrefs structure at CDGBegin()

time, are used to display the mode.

INPUTS

N/A

RETURNS

N/A

NOTES

The REW mode sprite(s) is displayed until CDGPlay() is called,

CDGBack() is called, or some other cdg.library mode function

is called.

cdg.library/CDGStop                                       cdg.library/CDGStop

NAME

CDGStop -- Tell CD+G that the disk is in STOP mode

SYNOPSIS

VOID CDGStop( VOID )

VOID CDGStop( VOID );

FUNCTION

Tell the cdg.library that the disk is in STOP mode.

Sprites, passed in a CDGPrefs structure at CDGBegin()

time, are used to display the mode.

INPUTS

N/A

RETURNS

N/A

NOTES

The STOP mode sprite(s) is displayed until CDGPlay() is called,

CDGBack() is called, or some other cdg.library mode function

is called.