The Bridge Card and Janus.Library

Robert J. Mical

27 April 1988

Copyright (C) 1986, 1987, 1988, Commodore-Amiga Inc.

All Rights Reserved

This reference document describes the capabilities and features the Bridge

hardware and of janus.library, the Amiga software library that supports

inter-processor communications between the Amiga and the Bridge PC.

Programming techniques are discussed, with examples provided to

illustrate the required techniques.

This document describes Version 2.0 of the janus.library.

This is a pre-beta release of this document.  You will notice that some

of the descriptions, especially about the lower system levels, are

sparse at best, and in some places non-existant.  On the other hand,

the autodocs and data structures are quite solid and can be depended on.

This document is comprised of the following sections:

- Introduction

- Services

- Janus Memory Requirements and Routines

- Other Support Routines

- Janus Signal Management

- Bridge/Janus Interrupts

- Amiga Function Call Specifications ("autodocs")

- Amiga Include Files

- PC Include File and Assembler Interface Specifications

- ARead, AWrite and JFTrans on the PC

- Brief Hardware Description

- Hardware Registers

=== ===========================================================================

=== Introduction ==============================================================

=== ===========================================================================

"Where's that confounded bridge?"

Led Zeppelin

The Amiga 2000's Bridge board provides PC compatiblity to the Amiga.

This is accomplished with a complete PC hardware environment on the

Bridge card combined with special card slots inside the A2000 to accomodate

PC hardware cards.

Except when they pause to communicate, both the Amiga and the PC are

able to execute at full speed without interfering with one another.

In this way, the A2000 can provide a true multi-processor environment,

which, while perhaps not as powerful as the multi-processor environment

of a Cray X-MP, has great potential indeed.

When the two machines need to communicate, they do so through a software

interface called "Janus", which name comes from the mythological god

with two faces (though some of the engineers would have named the interface

"Zaphod" after the popular-fiction character who had two heads which were

constantly bickering).  The Janus interface uses the Bridge board's

interrupt system and 128K of dual-ported RAM to provide these

inter-machine communications.  This RAM can be accessed by either the

Amiga's or the PC's processor.  It is a special sort of RAM that is not

generally available to Amiga programs, so you have to use the special

janus.library function calls to allocate and manipulate this memory.

Details of this are described in the "Janus Memory Requirements and

Routines" section of this document.

Software on the two machines can communicate on three levels.  At the

top is, as you might imagine, the most convenient level to use:  services.

Janus Service routines provide an easy way for Amiga and PC programs to

communicate.  The entire underlying support for these communications is

hidden from the programmer, requiring but a few standard Amiga and PC

programming techniques to send and receive signals and data between the

two CPU's.  Below the services routines are the Janus signal interrupt

routines, which code creates a level of interacting with the PC that

requires direct handshaking with the PC but hides the complexities of

interrupt handling from the programmer.  Beneath this level is the Janus

interrupt level itself.  Here is found the routines which allow you to

interact with the Bridge hardware on an interrupt basis.  Most programmers

don't even want to know about this level, let alone actually use it.

Besides, most of the hardware interrupts are spoken for.

To learn more about the three levels of system support, refer, in

descending order, to the sections "Services," "Janus Signal Management"

and "Janus Interrupts" of this document.

The Amiga programmer's interface and the PC interface are almost

identical.  There are a handful of differences which reflect the fact

that the Amiga multi-tasks, the Amiga doesn't allow busy-wait loops,

the Amiga doesn't allow hard-coded addresses for routines, and other

stuff like that there.  The interface to the PC routines can be found

in the "PC Include File and Assembler Interface Specifications" section.

A library of C-language-callable routines will be available soon.