Description and Usage of Bridge/Janus Dual-Ported Memory

Robert J. Mical

29 April 1988

Copyright (C) 1986, 1987, 1988, Commodore-Amiga Inc.

All Rights Reserved

This section describes the memory layout of the Amiga 2000 Bridge card

and discusses how to gain access to and use the features of this special

memory.  This section is divided into the following subsections:

- Introduction

- Memory Access Address Spaces (Defines Type)

- Memory Areas (Defines Type and Offset)

- Rules for Reading and Writing Janus Memory

- Locking and Unlocking Janus RAM

- The Inner Sanctum of Locks

- The Read-Only Write-Only Protocol

- Example:  Amiga/PC Traversing a Janus Memory Linked List

- The Memory Functions

=== Introduction ==============================================================

The Janus RAM is a special sort of RAM which can't be accessed in the

normal Amiga or PC way.  You need to make particular system calls to

access the memory, and you need to exercise certain techniques which,

though not too difficult, are nevertheless non-standard.  Also, you may

want to consider protocols which define ways for a program on the Amiga

and one on the PC to read and write to a shared data block without

destroying one another's efforts.  Such a protocol is called a "locking

protocol" and is defined below under "Locking and Unlocking Janus RAM."

The Janus hardware interface contains 128K of dual-ported RAM.  This RAM

can be accessed by either the Amiga's or the PC's processor.  The 128K

size of the RAM reflects today's reality; tomorrow there may be more,

perhaps much more.

Even though both CPU's can access the RAM, the bus structure of the RAM

is set up such that only one processor at a time can perform a memory

read or write.  Either processor, once beginning a read/write operation,

can be assured that the operation will complete before the other processor

gains access to the memory.  The implication of this is that it's not

possible for both processors to bang simultaneously on the same memory

location with a mixed result, or one processor reading a partial value

from a memory location that the other processor is in the process of

writing.  A benefit of this is that a safe alebit crude "lock" mechanism

can be implemented, which locking mechanism is described in the "Locking

and Unlocking Janus RAM" subsection below.  Also, it allows for a safe

and simple read-only write-only protocol, which allows programs to pass

data without locking.  This protocol is described under "Read-Only

Write-Only Protocol" below.

The total "address space" of the Janus memory, which is to say the

total number of memory locations that you can read to or write from

in Janus memory, looks much larger than the actual amount of Janus RAM.

This is because each physical byte of the Janus RAM can be read from

or written to using several different ranges of memory addresses.

The data contained in the RAM looks different depending on which address

you use to access it.  Normally, RAM looks like RAM, but with the Janus

memory you can also look at the RAM through an address space where the

bytes of each word are swapped, where the bits are all swizzled together,

where the RAM is scrambled.  You may wonder why you would want to read

scrambled RAM?  In the "Memory Access Address Spaces" section below the

RAM differences are described and an explanation and examples are given.

Each of these address spaces of the RAM has its own complete set of memory

locations, even though, as noted above, all the sets of memory locations

address the same physical RAM chips.

Also, the Janus memory is divided into area that are a maximum size of 64K.

Currently, there are only two areas of RAM that can be addressed:

parameter and buffer RAM.  Memory areas and memory "offsets" are discussed

in the "Memory Areas" subsection below.

You will find a value name "type" used in the discussions below and as

an argument to many of the janus.library functions.  The type value

describes both the memory address space that you are using and the memory

area of your RAM.  Sometimes you specify the type value, other times you

make system calls to learn the type of a block of Janus memory.  Also,

you will find the phrase well-formed type" used below.  This refers to

a type argument that correctly defines both the memory address space and the

memory area of the RAM.  Al well-formed type has descriptors for both.

=== Memory Access Address Spaces (Defines Type) ===============================

The "address spaces" of Janus memory refers to the ranges of memory

addresses that are available on the Amiga side when addressing Janus memory.

The data contained in the RAM looks different depending on which address

space you use to access it.

Note that the discussion of address space is for Amiga programmers only.

On the PC side, RAM always looks like RAM is expected to look.  The burden

is on Amiga programmers to adjust and translate memory pointers depending

on which type of memory operation will be performed.  The Amiga takes all

the weight of this; the PC, limping along as it does, needs all the help

it can get.

The simplest memory access address space is the byte-access address space.

Individual bytes of RAM are read and written through byte-access, which

is just like normal byte read and write operations in any Amiga or PC RAM.

But while byte reads/writes are the same on the two machines, word

reads/writes differ significantly.  On the one hand, when a word of memory

is written by the Amiga with its 68000 processor family, the high byte is

written first and the low byte is written at the next address.  On the PC

with its 8088 processor family, when a word of memory is written the low

byte is written first and the high byte is written at the next address,

which is exactly the opposite of the Amiga.  If the Amiga wrote a word

value to regular memory and the PC read that word, the value read would

have its bytes swapped (e.g. if the Amiga wrote $00FF to a memory location

and the PC read that location, the PC would find the value $FF00 in its

internal register).

To solve this inherent problem, Bridge provides a memory address space

called "word-access memory", where a word read into an internal register

by the Amiga looks the same as that word when read in by the PC.  If the

Amiga writes $00FF to word-access RAM, it looks like $00FF if the Amiga

then reads that same memory location.  But the bytes of the Amiga's word

are swapped on their way to real RAM, so that the RAM actually contains

the values $FF-$00 which, when read as a word by the PC, becomes the

value $00FF as desired.

This is a great advantage to programmers, who are freed of the obligation of

swapping bytes when reading words written by the other processor.

To specify byte-access address space in the type argument, you use the

constant MEM_BYTEACCESS.  To specify word-access address space, you use

MEM_WORDACCESS.

There is another memory access address space provided by the Bridge, the

graphics-access address space.  Graphics-access is not generally useful

to programmers.  This is an address space where the bits of a word are

scrambled in a way that makes reading graphics data from the PC convenient

on the Amiga.  On the PC, graphics data is laid out in a fashion where

the 2 bits of a pixel are packed together side by side in the same byte,

4 pixels per byte, whereas the bits of an Amiga's pixel are spread out

one per bitplane.  Reading PC graphics data and converting to Amiga graphics

is quite a complicated and slow affair.  The graphics-access memory space

provides a way of helping hasten this process.  In the graphics-access

address space, the bits of a word are shuffled up in such a way that the

first bits of each pixel will be packed together into the first byte of

a word and the second bits of each pixel will be packed together into the

second byte of the word.

For instance, here's what a word of graphics-access data

would look like on the PC and on the Amiga (in binary and hex):

PC:  01010101 01101001  $5569      00010010 00110100  $1234

- - - -  - - - -              - - - -  - - - -

Amiga:  00000110 11111001  $06F9      00010100 01000110  $1446

--------                      --------

Perhaps you agree that this isn't generally useful to Janus interface

programmers, unless of course you're bringing PC graphics into an Amiga

application, or vice versa (which is a strange but useful thing to think

about).

Another data access method that will come available in the future will be

the longword-access address space, which will allow the Amiga and a 386

device to read and write normal longwords (32-bit) to Janus memory.

As of this writing there is no 32-bit address space, so the management of

32-bit (or larger) quantities must be by a convention agreed upon by the

joint applications.

=== Memory Areas (Defines Type and Offset) ====================================

The Janus RAM is divided up into memory areas, each of which has a maximum

size of 64K.

Currently, there are two memory areas available:  buffer and parameter.

Buffer memory is 64K big, and parameter memory is 16K.  These sizes

are subject to change without notice.  When you allocate memory, you must

specify the memory area you want used for your allocation (it is an error

to not specify the memory area).  Most often programmers will allocate

memory out of the buffer area.  You may use any of the memory address

spaces when accessing any of the memory areas.

In the future, more memory areas will be made available.  Each memory

area will always be a maximum of 64K.

When using Janus memory, often you will be required to deal with the memory's

"offset." The Janus memory offset is the number of bytes that a particular

byte is displaced from the first byte of its memory area.  For example,

if a particular memory area of Janus memory turned out to be $200000 and

you were interested in the byte at $200010, then the offset to your byte

is $10.  Because a Janus memory area will never exceed 64K, the offset to

the memory will never exceed 16 bits.  In several of the Janus data

structures you will find notation "RPTR Offset" where RPTR stands for

"Relative PTR," which is defined as a 16-bit quantity used to define

an address that's relative to some memory base.

A note about a peculiar Janus memory idiosyncracy:  an offset of 0

into Janus memory is a valid offset, of course, which messes up a

standard that we've come to expect, which is that zero means "error."

In Janus memory, 0 is good.  The "not a valid offset" value is -1.  This

is the value you'll find when you expect either a valid offset or

"no offset defined."

=== Rules for Reading and Writing Janus Memory ================================

The freedom that comes with the memory-swizzling address spaces and areas

of Janus RAM does not come free:  the Amiga programmer is obliged to always

make sure to use the correct address space when accessing data, and both

Amiga and PC programmers must pay attention to the memory area that's

being used.

The rule for address space is simple, but you must remember to use it:

when reading individual bytes, read from the byte-access address space.

When reading words or longwords, use the word-access address space.

There are a group of routines on the Amiga which assist you in making sure

that you're addressing the correct address space for the job.  For

instance, JanusMemToOffset() creates a Janus memory offset from a regular

memory pointer, JanusMemType() creates a Janus memory type from

a regular memory pointer, and JanusOffsetToMem() creates a regular

memory pointer from an offset and a type.

By the way, don't let the memory complications put you off too much.

Once you have used the memory type and the offset to create a pointer to

the correct type of Janus memory, then you can use that pointer just like

any Amiga or PC pointer.

=== Locking and Unlocking Janus RAM ===========================================

If there's a possibility that both an Amiga program and a PC program might

be manipulating the same memory, you probably want to require the programs

to "lock" access to the memory before beginning.

A Janus lock is a software/hardware mechanism which can make a lock out of

any byte of Janus RAM.  Only one processor can "own" the lock at a time.

If one owns the lock and the other tries to get it, the other will have to

wait until the first releases the lock.

Why would you want a lock?  As an example, if you have a data structure

in Janus memory which can be written to by either the Amiga or the PC,

then each side should lock the other out from using this memory to

avoid problems where, for instance, both processors are trying to write to

the same word at the same time.  As another example, if you have a linked

list in Janus memory which can be manipulated by either the PC or the Amiga,

then you should lock access to this list before changing it.  Imagine if

the two processors could simultaneously link and unlink fields in the

list without regard to what the other is doing.  Sooner or later the guru

comes to play chess with you.

A lock:

- controls access to a specific area of memory which may be used

by more than one program but should be used by only one program

at a time

- only one program can own a lock at a time

- the lock protocol, described below, provides a way for programs

to attempt to own a lock and to wait until they do own the lock

- only the program that owns the lock can use the memory associated

with the lock

- the program that owns the lock knows that it can use the memory

without concern about possible interference from other programs

To create a Janus lock is rather easy.  All you need is a byte of Janus memory,

which you initialize with the value $7F.

The Janus library provides C-callable functions to get ownership of a Janus

lock (named JanusLock() and JanusLockAttempt()) and assembly-language

macros (named LOCK and LOCKTRY) which do all the work for you.  Your only

job is to remember to call JanusUnlock() (or UNLOCK as appropriate) as

soon as you possibly can.  Note that the lock is a byte of memory, so the

address you pass to the JanusLock(), JanusLockAttempt() and JanusUnlock()

functions must point to the Bridge board's byte-access memory space.

The JanusLock() function is designed for high-performance applications

where fast through-put is essential and where both Amiga and PC programs

promise to never own the lock for long.  To achieve this high-performance

lock, the Janus library, for lack of a better alternative, will commit

one of the great Amiga sins for you:  JanusLock() sits in a busy-wait loop

waiting for the lock byte to come available.  The PC-side lock does the

same thing, but on the PC, when you're rude, no one cares.

JanusLockAttempt() is the mellow version of the normal JanusLock() function.

JanusLockAttempt() tries once to get the lock and returns a BOOL TRUE or

FALSE if it succeeds or fails respectively.  This allows you, if your

program opens GfxBase, to create the MellowLock() function:

MellowLock(lockbyte)

UBYTE *lockbyte;

/* Go about getting a Janus lock in a mellow way, trying only 60

* times per second (or 50 times per second in some locations)

* to get the lock, calling WaitTOF() between calls to

* JanusLockAttempt().  Note that the following code presumes that

* you've opened graphics.library.

*

* When this routine returns, you own the lock.  Don't forget to

* call JanusUnlock().

*

* Remember that lockbyte is a Janus memory pointer and should be

* byte-access oriented.

*/

{

if (NOT JanusLockAttempt(lockbyte)) WaitTOF();

}

For you Service users, the ServiceData structure that you get the address

of has a lock-byte field, named, imaginatively enough, ServiceDataLock.

The lock is initialized by the system for you, after which your program

is free to use this lock in any way.  Most Amiga/PC service programs that

share memory will want to lock one another out before accessing the memory.

There is an AddService() Flag named ADDS_LOCKDATA which you can set to

specify that the ServiceDataLock should be set for you before your service

is added to the system.  This lock allows you to take initialization steps

such as preparing your parameter memory before users of your service will

have access to the contents of the ServiceData structure.  This presumes

that you have declared a protocol where users must lock the ServiceDataLock

before accessing the ServiceData.

Note that if you have more than one Amiga program going after the same Janus

lock they too must lock each other out.  Fortunately, the same locking

mechanism will work.  Unfortunately, the locking mechanism is a naughty

one because the only way to check whether or not you have gotten the lock

is by testing repeatedly, which is called a "busy-wait loop," which is

considered bad programming style in a multi-tasking environment.

Really really bad programming style.  Also, it's fairly dangerous, because

if there are two Amiga tasks conected to the same service, where one task

has a higher priority than the other and the lower-priority task has a

lock on memory when the higher-priority task decides it wants it, the

higher-priority task will busy-wait loop forever trying to acquire the

lock, never giving the lower task the time-slice it needs to be able to

relinquish the lock.  This is one example of a dreaded problem called

dead-lock, the only way out of which is with that familiar three-key

sequence.

An Amiga-approved way out of the dilemma is to use an Exec semaphore, but

there is some overhead involved with semaphores which can degrade

performance unnecessarily when you have a situation where good performance

is important and you know that any owner of the lock will never own it

for long.  Besides, if one application has what's needed by the PC but it's

prevented by satisfying the PC because a different task has owns the

semaphore, we're back to dead-lock.

A less complicated way to solve the problem is to require all Amiga

programs to use Forbid() to hold each other off while using the Janus

memory.  But this solution is extreme; one should avoid Forbid()/Permit()

whenever possible.  And besides, we can dead-lock here too.

A slightly more forgiving way to busy-wait loop, which helps solve the

problem of multiple Amiga programs needed to let each other have a

chance at satisfying the PC, is to test the lock and if you fail to

lock then call WaitTOF() before testing again.  At least this allows

the system to breathe in between your checks, although it causes you to

test the lock only 60 times per second, which is unacceptably slow in

high-throughput applications.  Also, this is a programming style that

some Amiga developers eschew but which others take advantage of regularly

because it provides a quick and easy way to wait for an event in a

multitaskingly-polite way.

Refer to the Janus library documentation for details regarding JanusLock(),

JanusLockAttempt(), JanusUnlock(), LOCK and UNLOCK.

=== The Inner Sanctum of Locks ================================================

This section purports to explain how locks work at the innermost level.

You do not need to understand the contents of this section in order to

use locks.  The intention of this section is as a tutorial, so you can

skip it if you want.  In fact, you probably want to skip this section.

Do not underestimate the power of the dark side, young programmer.

By definition, an "unlocked" lock byte contains the value $7F while

a "locked" lock byte contains the value $FF.  Although the lock byte

will always have these values, you will probably never have to know

about it because you never have to look at the whole value of the lock

byte, you look only at bit 7 (the most significant bit) of the lock byte.

If this bit is set, then someone else already owns the lock.  If this

bit is clear, you can own the lock if you set the bit.

The problem with implementing a safe lock mechanism is that you must be

able somehow to test for the availability of the lock and actually get the

lock all in a single step.  If testing for and getting the lock requires a

multi-step process, you can get into serious trouble because someone can

steal the bus from you halfway through your steps and the result can be

garbage.

Consider the multi-step case where to get ownership of a lock first you

read the lock byte, examine bit 7 to see that the bit is clear, and if it

was clear then set the bit in the lock byte to designate that now you own

the lock.  This multi-step process won't always work correctly if you

take more than one bus cycle to both examine the lock and mark it for your

own, because both your program and a program on the other side may be

racing simultaneously to get the lock and you may fool each other that

the lock is available to both, as the following example illustrates:

Real-time Amiga Events                 Simultaneous real-time PC events

------------------------------------   ---------------------------------------

Amiga wants to get a lock              PC wants to get a lock

Amiga asks for the Janus memory bus,   PC asks for the Janus memory bus, must

gets it, reads the lock byte           wait because Amiga has the bus

Amiga examines the lock byte value,    PC now gets the bus,

sees that bit 7 is clear               reads the lock byte

Amiga asks for the bus, gets it,       PC examines the lock byte value,

sets bit 7 of the lock byte            sees that bit 7 is clear

to designate that Amiga

owns the lock

PC asks for the bus, gets it,

sets bit 7 of the lock byte

to designate that PC

owns the lock

At the end of this example, both CPU's believe that they own the lock.

Soon comes the guru.

So the trick is to test for ownership of the lock and claim ownership of

the lock all in one bus cycle.  How can we do this?  Well, it requires

a bit of fancy dancing by both processors to create the effect.

On the 68000 side there's an instruction called TAS, which stands for

Test And Set.  This instruction, in one indivisible memory cycle, tests a

specified memory location for zero and negative and sets bit 7 of the

memory location.  The effect of this is that you can set bit 7 of a lock

and find out whether or not bit 7 was already set (by testing the N flag)

all in one stroke.  If bit 7 hadn't been set before (N flag clear after

the operation) then you have successfully obtained the lock because TAS

sets bit 7.  If bit 7 had been set before (N flag set after the operation)

then you have not obtained the lock; meanwhile, the fact that TAS sets

bit 7 doesn't hurt anything because it's merely setting a bit that's

already set.

[Note:  the 68000 TAS instruction as described above doesn't work in the

Amiga chip memory because the bus is granted to the Amiga chip DMA devices

halfway through normal 68000 cycles and it's possible for the DMA devices to

jazz with a memory location that your TAS is working with without you

knowing.  Fortunately for our application, Janus memory is not chip memory

so we can use this technique.  But this is a warning that you

assembly-language programmers should note.  It's funny how this stuff works.

TAS is "indivisible" except of course when you divide it.  Kind of like

life, eh?]

On the PC side, the PC is going to test whether or not bit 7 is set by

rotating the lock's value left, which will rotate the carry bit into

bit 0 of the lock, rotate the lock left by one bit, and rotate bit 7 of

the lock into the carry bit.  In preparation for attempting to get the

lock, the PC sets the carry bit so that a set bit will be rotated into

bit 0 of the lock.  The PC has an instruction prefix named LOCK which

forces the next read/modify/write CPU operation to have indivisible

access to the bus, which allows it to perform the above operation is

one swoop.  At the end, the lock byte will have the value $FF.  If the

byte had had the value of $FF before the operation, then it still has

the value $FF except that now the carry bit is set to designate to the

PC that the lock was not gotten.  If the carry bit is clear, then the

lock byte had had $7F, which means it was unlocked, and now it has $FF

to designate that it's locked.

=== Read-Only Write-Only Protocol =============================================

If you need to transfer data between the two processors, there is a safe way

to do so without requiring the CPU's to lock before reading and writing

shared memory locations.  This is the read-only write-only protocol.

This protocol takes advantage of the fact that both CPU's are able perform

an indivisible read/write in Bridge RAM.

=== Example:  Amiga/PC Traversing a Janus Memory Linked List ==================

The example C code below could be used by either the Amiga or PC to

traverse a linked list.  This is based on the fact that the RPTR offset

value of -1 means "no memory" in the Janus memory.

struct ExampleStruct

{

RPTR   OffsetToNext;

USHORT Field;

UBYTE  ByteField0, ByteField1;

};

RPTR   offset;

struct ExampleStruct *exptr, *exptr_BA, *exptr2;

/* Get the Janus memory offset to the first ExampleStruct pointed to by

* the argument named arg.

*/

offset = arg->FirstExampleStruct;

exptr = NULL;

/* We do the loop while offset doesn't equal -1.  An offset of -1 in

* Janus memory means "end of memory"

*/

while (offset != -1)

{

/* We know, by our own definition, that all of our ExampleStruct

* structures will be located in buffer memory.  We want to have a

* word-access pointer to our structure, so our type is

* MEMF_BUFFER | MEM_WORDACCESS.  Turn this plus the offset into

* a pointer to our structure.

*/

exptr = (struct ExampleStruct *)JanusOffsetToMem(offset,

MEMF_BUFFER | MEM_WORDACCESS);

printf("next field = %ld\n", exptr->Field);

/* Now turn the word pointer into a byte pointer before reading the

* structure's byte-sized fields.

*/

exptr_BA = MakeBytePtr(exptr);

printf("next byte1 = %ld\n", exptr_BA->ByteField1);

/* Get the next offset.  Note that we're reading a word, which is

* why we asked for MEM_WORDACCESS above.

*/

offset = exptr->OffsetToNext;

}

if (exptr)

{

exptr2 = (struct ExampleStruct *)AllocJanusMem(

sizeof(struct ExampleStruct), MEMF_BUFFER | MEM_WORDACCESS);

if (exptr2)

{

exptr->OffsetToNext = JanusMemToOffset(exptr2);

exptr2->OffsetToNext = -1;

}

}

=== The Memory Functions ======================================================

AllocJanusMem(Size, Type)

Allocates Janus memory

AllocJRemember(...)

AllocServiceMem(ServiceData, Size, Type)

Allocates memory, attaches it to the ServiceData list so that it can be

freed automatically when the service is deleted and removed from the

system

FreeJanusMem(Ptr, Size)

Frees memory

FreeJRemember(...)

FreeServiceMem(ServiceData, Ptr)

Frees memory that had been allocated using an AllocServiceMem() call

JanusMemBase(Type)

Returns address of first byte of memory type

JanusMemType(Ptr)

Returns type of memory that Ptr points to

TranslateJanusPtr(Ptr, Type)

Translates the Janus memory pointer from its current type to the new

type, returns the pointer to the new type memory

JanusMemToOffset(Ptr)

Derives an offset of the pointer from the beginning of the type of memory

to which it's a pointer.  This is a two-step process:  first it gets the

absolute offset depending on the memory-access type, and then it

figures out if this pointed to parameter or buffer memory (these are

two different areas of memory which aren't necessarily continguous)

and creates the offset from the beginning of the type of memory.

JanusOffsetToMem(...)

MakeBytePtr(...)

MakeWordPtr(...)