TABLE OF CONTENTS

MapBoard

MapBoard                                                             MapBoard

NAME

MapBoard - A 68040 Expansion Board ROM mapper

SYNOPSIS

MapBoard will let you map the ROM of an I/O board as cacheable

at specific addresses on 68040 systems.

FUNCTION

MapBoard will use the MMU to map sections of an I/O board's

address space as cacheable such that things like device drivers

can run in the CPU instruction caches.  No memory should be

used by this program.

This program only is useful for 68040 based systems running

V37.10 or later of the 68040.library.

All options are also available from icon tooltypes in either

project or tool icons.

INPUTS

MANUFACTURE/N	- The Manufacture autoconfig ID for the board

PRODUCT/N	- The Product autoconfig ID for the board

OFFSET/N	- The address offset from the start of the board

where caching should begin.  This is rounded up

to the nearest page address.

SIZE/N		- The length in bytes of the cacheable area.

This will be rounded to the page size as

to only cache pages that are completely

contained within the area from OFFSET to

OFFSET+SIZE

A2091/S		- Do the Commodore-Amiga A2091 hard drive

controller ROM.  This makes the A2091

run over 10 times faster on the A4000

when talking to non-DMA memory.

RESULTS

The MMU tables are modified to show the specific pages as

cacheable.  MapBoard does not (currently) produce any error

messages.

WARNING

Specifying invalid SIZE or OFFSET can cause the hardware device

to fail.  MapBoard does not any specific knowledge of the hardware

you are mapping (other than the A2091) and thus can not error

check your input.

NOTES

Since this tool modifies the existing MMU tables, toys that play

with the MMU can cause it to not work.  MapBoard will correctly

ignore invalid MMU tables as best it can.  The tables must be

4K page size and be fully valid 68040 tables.

Also, since Enforcer and such tools install a new MMU table

when they run, you will need to run MapBoard after starting

Enforcer.  If you quit Enforcer and then restart Enforcer

you will need to run MapBoard again.  If you run MapBoard

before Enforcer, Enforcer will correctly restore the MMU

settings when it quits.  So, if you ever do run Enforcer and

turn it off you may wish to have MapBoard run BEFORE you

run Enforcer and after each time you start Enforcer.

SEE ALSO

"A master's secrets are only as good as the

master's ability to explain them to others."  -  Michael Sinz

BUGS

None?