AutoBoot Documentation

by Bill Koester

AutoBoot is system software that allows the bridgeboard to use an Amiga

file as a PC hard drive. The fake hard drive is emulated at the PC's BIOS

level for 100% compatability. To the PC the AutoBoot volume appears as a

normal PC hard drive. On the Amiga side the hard drive exists as an AmigaDOS

file on any device and directory you choose. Any size PC hard drive can be

emulated as long as there is enough room for the Amiga file. This AutoBoot

file can then be used as a normal PC hard drive, it can be fdisk'ed,

formatted, and made to boot the PC at power up.

SETUP:

------

To set up an autoboot volume you need to do the following:

1). Edit your startup-sequence to include this line after the bindrivers

command.

run >nil: sys:pc/pcdisk

AutoBoot uses pcdisk to access the Amiga file so pcdisk must be

running when the PC begins its boot procedure.

2). Create the AutoBoot file on the Amiga. To do this use the makeab

command included with the PCInstall disk. Decide where you want the

file to be and issue the command

makeab  drive:directory/filename

makeab will now ask you for the number of heads, sectors/track, and

number of cylinders to emulate. AutoBoot can emulate any type and

size of PC hard drive, even hard drives that do not really exist!

To set up a 10 meg fake drive using an Amiga Hard drive you could

give makeab the following parameters:

makeab  dh0:fake_c

enter number of heads:     4

enter sectors/track:       17

enter number of cylinders: 306

makeab will now print the size of the proposed file and ask you

to accept it. You can now either restart and choose different

parameters, or create the file. If you accept the parameters makeab

will generate the file in the proper format for AutoBoot. Note:

makeab creates the file FULL-SIZED and may take a while to create

large volumes.

3). Now that the file is created, you must tell the system where to find

it. The system looks for the the file ABOOT.CTRL in the SYS:PC/System

directory. This file should contain the full path-filename of the

file you created with makeab. For this example you would use ed

or a suitable editor to create the file sys:PC/System/aboot.ctrl and

in this file there should be one line followed by a carriage return

dh0:fake_c

which is the name of the file just created. Save this file and you

are now finished with work on the Amiga side. Note: You can have

multiple AutoBoot volumes and switch between them by changing the

contents of ABOOT.CTRL, only one AutoBoot volume can be active at

any one time and you must reboot the Amiga to make the change active.

USE:

----

To use the AutoBoot volume from the PC you will first need to boot the

PC with a standard DOS disk. Follow the normal procedure for initializing a

PC hard drive, ie fdisk and format.

First run fdisk on the PC. Then partition the fake hard drive, you will

normally use the entire disk for the DOS partition, selection 1, but

you may also partition the hard drive in any way you like.

Next format the fake drive using the PC format command. IF YOU WISH TO

MAKE THE DRIVE BOOTABLE YOU MUST USE THE /S OPTION WITH FORMAT. Ex

Format c: /s

You may now remove the floppy DOS disk and reboot the PC. The PC will now

boot off the AutoBoot volume and you may copy files onto it and use it

as a normal PC hard drive.

NOTE:

-----

If you have a real PC hard drive and controller installed AutoBoot will

be disabled!

If your AutoBoot file resides on a FastFilingSystem partition you MAY need

to add the following line to the partitions mountlist entry

Mask = 0

Since this will slow down all file access to the partition it is recomended

that you create a separate partition for the AutoBoot file.