DEFINITION OF SYS:PC/SYSTEM/2500PREFS FILE

BYTE 0

BIT 7

must be 1

BITS 6,5

00 = illegal

01 = load Janus Handler to segment A000

10 = load Janus Handler to segment D000

11 = load Janus Handler to segment E000

BIT 4

0 = CGA emulation disabled

1 = CGA emulation enabled

BIT 3

0 = MDA emulation disabled

1 = MDA emulation enabled

BIT 2

must be 1

BIT 1

must be 1

BIT 0

must be 0

BYTE 1

BIT 7

0 = speaker on

1 = speaker off

BIT 6

0 = boot with MDA

1 = boot with CGA

BIT 5-3

000 = A: internal, B: internal

001 = A: internal, B: shared

010 = A: internal, B: external

011 = A: external, B: internal

100 = A: shared,   B: internal

101 = A: external, B: shared

110 = A: shared,   B: external

111 = illegal

NOTE: 000, 010, 011 require that Flipper be placed

in Amiga mode.

BIT 2

0 = sharing DF0:

1 = sharing DF1:

NOTE: only used if BIT 5-3 indicates a "shared" drive in use

BIT 1

0 = sharing a normal (880K) drive

1 = sharing a dual speed (880K/1.4M) drive

NOTE: only used if BIT 5-3 indicates a "shared" drive in use

BIT 0

0 = do not load shadowable Janus Handler

1 = load shadowable Janus Handler