From hedley@crunch Thu Nov 12 15:58:19 1992

Received: by cbmvax.cbm.commodore.com (5.57/UUCP-Project/Commodore 2/8/91)

id AA14308; Thu, 12 Nov 92 15:58:16 EST

Date: Thu, 12 Nov 92 15:57:06 EST

From: hedley@crunch (Hedley Davis -- Systems Engineering)

Received: from slug.lsidesign (slug.ARPA) by crunch (4.1/3.1.090690-Commodore International Services Corporation)

id AA11213; Thu, 12 Nov 92 15:57:06 EST

Message-Id: <9211122057.AA11213@crunch>

To: jerryh@cbmvax

Status: R

NEED TO ADD CACHE CONTROL BIT

Rev History:

11/2/92 Hedley  Reordered BITS register.

Add IOPORT.

10/30/  Hedley  Added CDTXDMA bit

Added Zero bit define for terminus word

clipped PBX to 16 bits.

10/14/  Hedley  Minor memory map changes...

10/9/92 Hedley  Added CDROM PBX explaination for jerry.

9/25/92 Hedley  still more cleaning..

9/12/92 Hedley  more cleanup based on better understanding.

8/30/92 Hedley  Added IR registers..

8/20/92 Hedley  Added command control DMA regsiters

8/13/92 Hedley  Cut 2

8/11/92 Hedley  First cut

NOTE:

Due to a modification of drive hardware to reduce the command

control interface to three wires, we will need to implement

both the new and old low level CDROM command control interfaces.

Selection of mode will be hardware rev.

68020 memory map

________________

For all function codes 0,3,4, or 7:

Create buss error combinatorially from AS_.

( Have clock delay on leading edge ).

For function codes 1,2,5 & 6:

addr         size    waits   Function

--------------- --      -----   -----------

$000000-$1FFFFE 32      agnus   Chip Ram        *1

$200000-$B7FFFE 16      Zorro   AutoConfig Space

$B80000-$B87FFE 16      none    internal registers

$B88000-$B8FFFE 16      1186    M1186 local processor interface.

$B90000-$B9FFFF 16      agnus.  Unmolested custom chip registers...

$BA0000-$BDFFFE 16      Zorro   unused

$BE0000-$BEFFFE 16      8520    Always present access to internal 8520s.

$BF0000-$BFCFFE 16      8520    Illegal 8520 access /  unsupported

$BFDr00         16      8520    8520A   Legal decode for 8520 A register r.

$BFDr02-$BFDrFC 16      8520    Illegal space that may or may not hit 8520 A.

$BFDrFE         16      8520    8520A   Legal decode for 8520 A register r.

$BFEr00         16      8520    8520B   Legal decode for 8520 E register r.

$BFEr02-$BFDrFC 16      8520    Illegal space that may or may not hit 8520 B.

$BFErFE         16      8520    8520B   Legal decode for 8520 B register r.

$BFF000-$BFFFFE 16      8520    Illegal 8520  access / unsupported

$C00000-$DEFFFE 16      Zorro

$DF0000-$DFFFFE 16      agnus   RGA     The one true location for Amiga chips.

$E00000-$E7FFFF 16      ROM

$E80000-$F7FFFE 16      Zorro   Empty.

$F80000-$FFFFFE 16      ROM     16 bit internal ROM

DMA CHANNELS:

-------------

There are internal DMA channels.

CDROM   CDROM data channel

CDCOM   CD drive command/control receive channel.

SUBCODE Subcode data channel.

IRRX    IR receive data channel.

IRTX    IR transmit data channel.

SUBCODE DMA CHANNEL:

--------------------

The subcode DMA channel is strictly used for DMAing subcode

information into host memory. It has an unusual addressing scheme

designed to facilitate subcode processing.

Basically there is a base page pointer which dictates a 256 byte

region which subcode dma can occur to. This base page register

is write only, and never changes except when written.

Within the base page are two 128 byte areas of which 98 bytes

are considered significant. These regions form a double buffer

for subcode information. Hardware detects subcode frame syncs,

and causes data to be dmad to one buffer then the other switching

on a per frame basis.

When frame syncs do occur, the last received data is dumped to

memory. Because frames are 98 bytes long, the longword is not

full. In this case, bogus data is simply written on the extra bytes.

The actual address of the last byte written can be derived from

the subcode status register.

DRIVE COMMAND/CONTROL DMA CHANNEL

---------------------------------

The CDROM drive has a command and control DMA channel for data

from the drive only. ( Data to the drive can be written one to

four bytes at a time via the CDTx register ).

The receive operation is limited to a single 256 byte region of

memory. This region is set via the RXAd register.

An address comparitor is provided to allow the software to control

how many bytes of data must be received before the creation of

an interupt. This location is RXICmp.

The intent is that the host software sets the interface to

interupt on the third byte from the drive. ( Most packets are

three bytes long ). When the interupt comes, software can

determine if packet length. If three bytes, job is done. If

more than three bytes, software can set RXICmp to create interupt

when last byte of packet is received. Thus software has one

one to two interupts per received packet.

IR DMA CHANNEL

--------------

The IR DMA channel supports both receive and transmission of

Infrared signals.

For receive, the channel DMAs bytes of information corresponding

to IR edges into a 256 byte circular buffer memory. These bytes

correspond to edges of the received IR signal after demodulation.

Details on the register spec can be found in /project/cdg/spec/ir.

CONTROL REGISTERS

-----------------

A region of memory is carved out to contain several game machine

specific registers. These are detailed below. These registers

are typically of length 32 bits.

The registers are detailed below:

addr    name    func

----    ------  --------------------------------------------------------

d00      Revision Register.

B31:0   Read chip revision number...

( likely to increment often w/ early FGPA

prototypes ).

The proposed format is YYMMDDTT where

YYMMDD is the year month day of when

the logical specification was

created.

TT is a Commodore assigned technology

code indicating the embodyment of

the data ( XYLINX/ACTEL/HP GA/

LSI STD-CELL etc... ).

d04      Status: Status Register         ( read only ).

d08     Int2En                          ( read/write ).

d06     Unused...

These 3 registers provide interupt control.

Status indicates which parts of the chip may be requesting

interupt service.

Int2En and indicate which bits in status can cause an Int2 interupt.

The bits are orginized as follows:

BIT:    Function:               Cleared By:

a       B31:    Subcode Buffer Change   Read Subcode Addr Buffer latch.

d       B30:    TxD Buffer Empty ( ro ) Write CDTXDATA data register.

ad      B29:    RxD Buffer Full.        Read CDRXDATA register.

a       B28:    RxD Compare Register    Write CDRXCOMP  compare register.

d       B27:    TxD Tx DMA empty.       Change TxD pointer.

a       B26:    CDROM changed PBX:      Write CDROM PBX.

d       B25:    CDROM hung.             Reseek.

a10:    CDROMHIGH:              write only.

This write only register contains the high order CDROM

rom data address. Only bits 23:17 are significant.

a14:    DMAHIGH:                write only.

This write only register contains the high order 14 bits

of address for the the IR receive page, the CDRX page,

and the SUBCODE page. Only bits 23:10 are significant.

The Subcode page is this address w/ B9:8 = 00.

The CDRX    page is this address w/ B9:8 = 01.

The CDTX    page is this address w/ B9:8 = 10.

a18:    SubAddr:                Read only.

This is the page buffer pointer containing the

low order 8 bits of information assocated with the

buffer address of the last byte of subcode information

written to the buffer by the DMA hardware.

Only the low order 8 bits are significant.

a19:    CDRXAddr:               Read only.

This is the page buffer pointer indicating the

address of where the next byte of received CDROM

command channel data would be written two.

a1A:    CDTXAddr:               Read only.

This is the page buffer pointer indicating the

address of where the next byte of received CDTX data will be

sent from.

a1C:    RXICmp  Write only. Eight bits.

This register is used to ease interupt overhead of

CDROM communications. When the low order buffer pointer

for drive receive DMA equals, or passes this value, a

status bit is set. This status bit is cleared by writing

this register.

a1D:    TXICmp  Write only. Eight bit.

This register is used to point to the Nth plus one byte

for CDROM transmit communications.

a20:    CDROM PBX:              read/write.

This is a read write register.

It contains the CDROM PBX register explained elsewhere.

ad24:   BITS:   Various control bits for the hardware:  read/write.

a       31      SUBCTL  0       Subcode DMA is off. Init.

a                       1       Subcode DMA enabled.

a       30      CDTXD   0       Drive Command control TX via prog IO.

a                       1       Command control TX via DMA.

a       29      CDRXD   0       Drive Command RX via progio.

a                       1       Drive Command RX via DMA.

a       28      DCASEN  0       Double cas for ROM DMA to chip ram disabled.

a                       1       Double cas for ROM DMA to chip ram enabled.

a       27      PBXEN   0       PBX disabled, and cleared.

a                       1       PBX enabled.

a       26      ROMEN   0       Rom data path disabled & RESET.

a                       1       Rom data path enabled.

a       25      ROMRAW  0       SECTOR controller in ROM mode.

a                       1       SECTOR controller in RAW mode.

a       24      ROMMSB  0       CXD2500Q LSB first

a                       1       CXD2500Q MSB first.

d28:     CDTXRX  This is the CD UART register.

WRITES:

One to four bytes of data may be written to this register.

Once written, the register may not be written again until

it is empty. This is indicated to software via the TXEmpty

flag.

RECEIVES:

This register returns one byte of data from the drive

if there is a byte waiting ( see status register ),

and if the com interface is in RAW mode. If the

interface is in DMA mode, this register returns

undefined garbage.

a2C:    PortDat

This is an eight bit general purpose IO port.

Only two bits ( 7:6 ) are implemented.

This is the direction register for a general purpose

IO port.

Reads always return value at pin.

a2E:    PortDir

This is the data direction register for PortDat.

Custom Chip Address Notes:

__________________________

In order to support IR receivers, some liberities must be taken

with the custom chip registers that support IO devices.

The kludge is as follows:

Software that totally takes over the system ( to the point

of not allowing the keyboard driver to execute ) will not

be able to take advantage of the IR receiver.

Software that at least allows the keyboard process to execute

will be able to take advantage of IR recievers.

Software in the host will examine the FIRE button for IR events.

( in reality, IR events are pretimed by the hardware and an

event list is built in ram via hardware. Software needs to examine

the event list for IR smelling events. ).

If the software detects IR events, then software will want to

somehow cause applications software to think that mouse

buttons are depressed, the mouse has moved, pot events have

occured, and keyboard events shall have happened. A variety

of stratgies are to be employed to trick applications into

seeing the correct events.

1)      Keyboard events:

Keyboard events are simply 'fixed' in software.

This is easy because it is presumed that 99% of

developers realized banging the metal for keyboard

events was not a good plan. ( Bryce, et al ).

2)      Fire Button Events:

Special hardware is provided to make the 8520s think

that the fire buttons are pressed.

3)      Mouse/Pot events:

Mouse and pot events are created by remmapping some

custom chip registers into hardware. The software can

then control what other software sees when these registers

are read.

The magic remapped registers are:

Joystick:

JOY0DAT DFC00A  R

JOY1DAT DFC00C  R

JOYTEST DFC036  W

The two read locations are internally replicated as

two 16 bit registers that can be read and written by

system software. Joytest writes also affect these

two registers in the expected fashion.

Pots:

POT0DAT DFC012  R

POT1DAT DFC014  R

POTGOR  DFC016  R

POTGO   DFC034  W

The read locations are emulated in hardware via three

16 bit writeable locations. Writes to PotGO exhibit

the expected effects on the read locations.

This remapping can be controlled. It is normally off, assuming that

normal joysticks are connected. If enough system software is running,

and if this software sees bonafide IR events, then it can enable the

remapping to allow it to control the input stream of the application.

CDRROM DMA:

There are four distinct pieces of hardware involved in CDROM

DMA. These are the CDROMHIGH register, the PBX, the

SECTOR controller/DECODER. Each of these hardware devices is

responcible for a different portion of the destination

address.

CDROMHIGH:

----------

The CDROMHIGH register is a write only 7 bit register.

It contains the high order 7 bits of the address. This

contrains all CDROM dma to the same naturally aligned

128K byte region of memory.

PBX:

----

The PBX is a 16 bit read/sort of write memory location. Each bit in

the PBX corresponds to one 4K region in the 64Kbyte CDROMDMA

region. Bit zero corresponds to the low order sector buffer

as pointed to directly by CDROMHIGH, Bit one corresponds

to that address + 4k, etc....

Initially, at system reset, the PBXEN bit in the BITS register

is cleared. This forces the PBX to all zeros. In order to

to CDROM DMA, software must set bits in the PBX. Once a

bit is set in the PBX, system software cannot clear it with

out reseting the entire PBX. Set bits signify buffers which have

been 'given' to the hardware for CDROM DMA.

When the CPU writes to the PBX, bits with a value of zero have

no effect on the register. Bits with a value of one, forcibly

set the corresponding PBX bit. Once a bit is written with a

one, the host should never rewrite that bit with a one ( the

bit belongs to hardware ), until the bit is cleared by the

hardware ( the bit belongs to the software ).

When DMA needs to occur, the hardware picks a random set bit in

the PBX and uses this bit to create the address for the CDROM

DMA. When the sector is completed, the hardware clears the

corresponding bit in the PBX.  Software, by maintaining a copy

of the PBX, and xoring it against the current PBX value, can

easily determine which sector buffers have been returned by

the hardware. Software then must examine the contents of the

returned buffers to determine their validity, and which buffers

contain which sectors.

An interupt facility is provided to interupt the CPU when a

sector is returned to software. This shows up in the interupt

and status registers. The interupt is cleared by any write to

the PBX ( including one of all zeros which changes no bits

in the PBX ).

SECTOR/DECODER:

---------------

The sector controller is responcible for the actual decoding of

the CDROM data, performing a EDC check, and for DMAing the

the data into the destination sector as selected by the PBX.

The following bits control this section:

ROMMSB: 0       data is LSB first.

1       data is MSB first.

This bit corresponds to an analogous control bit

in the M1186. The hardware requires that this bit

be set to the correct value before taking the ROMEN

high, and left alone until after ROMEN is low.

ROMCLKR 0       data is clocked in on falling edge of BCLK.

1       data is clocked in on rising egde of BCLK.

This bit corresponds to an analogous control bit

in the M1186. The hardware requires that this bit

be set to the correct value before taking the ROMEN

high, and left alone until after ROMEN is low.

ROMRAW  0       Sector controller in ROM mode.

Sector controller in RAW mode.

ROMEN   0       Sector controller in reset & off.

1       Sector controller on, and running.

To perform CDROM DMA, the ROMHIGH regsiter, and the PBX

must both be set up. Then the ROMMSB, ROMCLKR and ROMRAW bits

must be set up. Then the drive should be made to start

spitting out data. Then ROMEN should be set. At this point

the hardware starts reading CDROM sectors, and DMAING them

into memory. Note that the only way to stop the hardware

from grabbing sectors is to take ROMEN low.

The following status are provided from this section:

CDROMHUNG:

0       Everything is cool.

1       CDROM data path failed to complete

DMA in time. Overflow. CDROM datapath

locked up. Reset is only fix. ( Probably

to much DMA in system for CDROM dma to

be able to occur ).

All other information is contained in the returned sector.

SECTOR FORMAT ( rom mode ):

---------------------------

Each sector of rom data is formated as follows:

Addr    Contents:

----

x000    Status Word

x00C    CDROM header.

x010    Start of CDROM data.

xC00    Start of C2P0 data.

The CDROM header is always DMAd first. It is the four bytes

of standard CDROM header info.

Starting at x010, the CDROM data is always DMAd into memory

in sequential order. A sector is 2352 bytes long consisting

of 12 bytes of sync, a four byte header, and 2336 bytes of

other stuff. These 2336 bytes of stuff is dma'd into x010

through x92F.

At the same time that the CDROM data is read in, the C2P0 information

is also read into a bit map starting at xC00. The low order bit

at xC00 corresponds to the first sync byte, and bits continue

upwards in 32 bit chunks. There is one bit for each CDROM byte,

and therefore there should be 2352/32 = 73.5 words of C2P0

information. Only the 73 whole words are DMAd here.  Note that

the alignement has been carefully controlled to cause bit positions

in this region to naturally point to byte positions at an offset

of 0 from the start of the sector buffer. ( 0x00 ).

The final word DMAd into memory is the terminus word. It contains

lots of useful information and is always DMAd to the zeroth location

in the sector buffer. ( This should make it easy enough to find ).

The terminus word is defined as follows:

B31:16  The last 16 bits of C2P0 information.

B15:    Always zero.

B14:10  Undefined. Garbage.

B9?     set if sync was inserted.

B8?     set if short sector ( unexpected sync ).

B7?     set if EDC error for CDI format detected.

B6?     set if EDC error for normal format detected.

B5?     C2 summary. Set if any C2 bits were set in the sector.

B4:0    Sector count since CDROMEN was set.

Bits marked with a '?' may move.

Note that because the order in which bits are removed from the PBX

is undefined, when software finds several sectors have been returned

to it via the PBX, the only way to determine the order in which

sectors were returned is to examine the status word at the start of

each Sector.

SECTOR FORMAT ( raw mode ):

---------------------------

This is currently imprecisely defined as follows:

Each returned sector will contain:

2352 bytes of data corresponding to a sectors worth of

music data starting at an offset of x00C.

73 words of C2PO information starting at xC00. ( NOTE:

the offset of these bits relative to the data at

0x00C is broken here. These bits are relative to 0x00C,

not 0x000. ).

A terminus word at zero of which only the C2 information,

the C2 summary, and the sector count are valid.

There is no defined relationship between the sectors returned

and 'real' audio sectors. When you turn the device on in RAW mode,

it simply grabs 2352 bytes of audio per sector, and hands it off.

This is because there is no sync information in audio data.

M1186DMA:

---------

DMA from the 1186 is handled as follows:

There are two control bits and a single address pointer for

the 1186.

The address pointer is a full 24 bit write only register located

at the same location as CDROMHIGH.

The bits are located and described in the BITS register above.

Note that enabling both the 1186, and any of the normal

CDROM stuff results in undefined unpredictable and undesirable

behaviour.

To do that DMA stuff:

M86EN, M86F <= 00.      ( set up internal hardware ).

Write CDROMHIGH with the 24 bit address for the target.

M86EN <= 1 .            Enable internal hardware.

Make the 1186 do however many bytes of DMA you want.

M86F <= 1.              Flush the last bytes out.

Wait some indeterminate amount of time for data to flush.

done.