System Check

The System Check screen is a diagnostic display triggered by

failures during the autoconfig process.

NOTE: It is currently possible to 'break' into the System

Check screen by holding down both mouse buttons during the

boot cycle.  This will be taken out before the final

release, or maybe it won't.

_#T_#h_#e__#D_#i_#s_#p_#l_#a_#y

The display consists of a red screen with the title "System

Expansion Board Check", a list of expansion devices

currently installed, and a continue button.

The expansion device list has one line for each device

found. each line contains the following fields.

Status         The status of a device can be "OK", or "BAD"

if the device failed in some way. Usually the

bad device will be a memory board that has

gone bad.

Manufacturer   This is the Commodore assigned Manufacturer

number for the expansion device.

Product        This is the manufacturer defined product ID.

Chain          If two or more devices exist on the same

board there will be lines drawn at the end of

the 'Product' field to indicate this.

The list of expansion devices is sorted so that the top

entry on the screen corresponds to the board closest to the

CPU. This is the rightmost board as you face an A2000 and

the device directly plugged into an A1000 in a possibly

chained configuration. I don't know where it is on an A3000.

Remember that chained expansion devices are all on the same

physical board.

NOTE: Some early A2091 cards were flawed and do not appear

to be chained.

To continue with the boot process with the device disabled

click on the 'CONTINUE' button with the mouse.

_#I_#m_#p_#l_#e_#m_#e_#n_#t_#a_#t_#i_#o_#n

System Check is implemented as the syscheck ROM-module. It

has a ROM-TAG priority such that it initializes after

expansion.library, graphics.library, and most of the other

ROM-modules, but before the expansion boards Diag code is

executed and the boot process has begun.

On startup syscheck checks the Flags field in ExpansionBase

for the flag EBF_BADMEM. If set syscheck opens a View and

generates the display based on the list of ConfigDev

structures in ExpansionBase. The flags in each ConfigDev

structure are checked against CDF_BADMEMORY to determine

which device failed.