trackdisk.device/TD_GETDRIVETYPE               trackdisk.device/TD_GETDRIVETYPE

NAME

TD_GETDRIVETYPE - return the type of the disk drive to the user

FUNCTION

This routine returns the type of the disk to the user.

This number will be a small integer.  It will come from

the set of DRIVE... defines in trackdisk.h

or trackdisk.i.

The only way you can get to this call is if the trackdisk

device understands the drive type of the hardware that is

plugged in.  This is because the OpenDevice call will fail

if the trackdisk device does not understand the drive type.

To find raw drive identifiers see the disk resource's

DR_GETUNITID entry point.

IO REQUEST

io_Command	TD_GETDRIVETYPE

RESULTS

io_Actual	the drive type connected to this unit.

SEE ALSO

TD_GETNUMTRACKS

trackdisk.device/TD_GETNUMTRACKS               trackdisk.device/TD_GETNUMTRACKS

NAME

TD_GETNUMTRACKS - return the number of tracks on this type of disk

FUNCTION

This routine returns the number of tracks that are available

on this disk unit.  This call obsoletes the older NUMTRACKS

hard coded constant.

IO REQUEST

io_Command	TD_GETNUMTRACKS

RESULTS

io_Actual	number of tracks accessible on this unit

SEE ALSO

TD_GETDRIVETYPE

trackdisk.device/TD_RAWREAD                         trackdisk.device/TD_RAWREAD

NAME

TD_RAWREAD - read a raw sector from the disk

FUNCTION

This routine performs a raw read for the track disk.

It seeks to the specified track and reads it in to the

user's buffer.  This buffer MUST be in chip memory.

NO PROCESSING OF THE TRACK IS DONE.  It will appear exactly

as the bits come out off the disk -- hopefully in some legal MFM

format (if you don't know what MFM is, you shouldn't be using

this call...).  Caveat Programmer.

This interface is intended for sophisticated programmers

only.  Commodore-Amiga may make enhancements to the disk

format in the future.  We will provide compatibility

within the trackdisk device.  Anyone who uses this routine

is bypassing this upwards compatibility.  If your application

breaks, TOUGH!

If this warning is not enough, then add suitable additional

harrassment of your choice.

IO REQUEST

io_Flags	if the IOTDB_INDEXSYNC bit is set then the driver

will make a best effort attempt to start reading

from the index mark.  Note that there

will be at least some delay, and perhaps a great

deal, of delay (if, for example, interrupts have

been Disabled()...).

io_Command	TD_RAWREAD or ETD_RAWREAD.

io_Length	Length of buffer (in bytes).  The maximum allowable

length is 32K bytes.

io_Data		Pointer to buffer in chip memory where raw track

will be read into.

io_Offset	The track number to read in (not this is different

from a normal trackdisk io call which is given

in terms of logical bytes from the beginning of

the disk.  This is because the trackdisk driver

has no idea what the format of the disk is).

iotd_Count	(ETD_RAWREAD only) maximum allowable change counter

value

RESULTS

io_Error	non-zero if there was an error

LIMITATIONS for synced reads and writes

There is a delay between the index pulse and the start of bits

coming in from the drive (e.g. dma started).  This delay

is in the range of 135-200 micro seconds.  This delay breaks

down as follows: 55 microsecs is software interrupt overhead

(this is the time from interrupt to the write of the DSKLEN

register).  66 microsecs is one horizontal line delay (remember

that disk io is synchronized with agnus' display fetches).

The last variable (0-65 microsecs) is an additional scan line

since DSKLEN is poked anywhere in the horizontal line.  This leaves

15 microsecs unaccounted for...  Sigh.

In short, You will almost never get bits withing the first 135

microseconds of the index pulse, and may not get it until 200

microseconds.  At 4 microsecs/bit, this works out to be between

4 and 7 bytes of user data of delay.

SEE ALSO

TD_RAWWRITE

trackdisk.device/TD_RAWWRITE                       trackdisk.device/TD_RAWWRITE

NAME

TD_RAWWRITE - write a raw sector to the disk

FUNCTION

NO PROCESSING OF THE TRACK IS DONE.  The disk will appear exactly

as the bits come out of memory -- hopefully in some legal MFM

format (if you don't know what MFM is, you shouldn't be using

this call...).  Caveat Programmer.

NO PROCESSING OF THE TRACK IS DONE.  It will exactly

as the bits come out off the disk.  Caveat Programmer.

This interface is intended for sophisticated programmers

only.  Commodore-Amiga may make enhancements to the disk

format in the future.  We will provide compatibility

within the trackdisk device.  Anyone who uses this routine

is bypassing this upwards compatibility.  If your application

breaks, TOUGH!

If this warning is not enough, then add suitable additional

harrassment of your choice.

IO REQUEST

io_Flags	if the IOTDB_INDEXSYNC bit is set then the driver

will make a best effort attempt to start writing

from the index mark.  Note that there

will be at least some delay, and perhaps a great

deal, of delay (if, for example, interrupts have

been Disabled()...).

io_Command	TD_RAWWRITE or ETD_RAWWRITE.

io_Length	Length of buffer (in bytes).  The maximum allowable

length is 32K bytes.

io_Data		Pointer to buffer in chip memory where raw track

will be read into.

io_Offset	The track number to read in (not this is different

from a normal trackdisk io call which is given

in terms of logical bytes from the beginning of

the disk.  This is because the trackdisk driver

has no idea what the format of the disk is).

iotd_Count	(ETD_RAWWRITE only) maximum allowable change counter

value

RESULTS

io_Error	non-zero if there was an error

LIMITATIONS for synced reads and writes

There is a delay between the index pulse and the start of bits

going out to the drive (e.g. write gate enabled).  This delay

is in the range of 135-200 micro seconds.  This delay breaks

down as follows: 55 microsecs is software interrupt overhead

(this is the time from interrupt to the write of the DSKLEN

register).  66 microsecs is one horizontal line delay (remember

that disk io is synchronized with agnus' display fetches).

The last variable (0-65 microsecs) is an additional scan line

since DSKLEN is poked anywhere in the horizontal line.  This leaves

15 microsecs unaccounted for...  Sigh.

In short, You will almost never get bits withing the first 135

microseconds of the index pulse, and may not get it until 200

microseconds.  At 4 microsecs/bit, this works out to be between

4 and 7 bytes of user data of delay.

SEE ALSO

TD_RAWREAD