TABLE OF CONTENTS

card.resource/BeginCardAccess

card.resource/CardAccessSpeed

card.resource/CardChangeCount

card.resource/CardForceChange

card.resource/CardInterface

card.resource/CardMiscControl

card.resource/CardProgramVoltage

card.resource/CardResetCard

card.resource/CardResetRemove

card.resource/CopyTuple

card.resource/DeviceTuple

card.resource/EndCardAccess

card.resource/GetCardMap

card.resource/IfAmigaXIP

card.resource/OwnCard

card.resource/ReadCardStatus

card.resource/ReleaseCard

card.resource/BeginCardAccess                   card.resource/BeginCardAccess

NAME

BeginCardAccess -- Called before you begin credit-card memory access

SYNOPSIS

result=BeginCardAccess( handle )

d0			a1

BOOL BeginCardAccess( struct CardHandle * );

FUNCTION

This function should be called before you begin access

to credit-card memory.

Its effect will depend on the type of Amiga machine your

code happens to be running on.  On some machines it

will cause an access light to be turned ON.

INPUTS

handle - Same handle as that used when OwnCard() was called.

RETURNS

TRUE if you are still the owner of the credit-card, and

memory access is permitted.  FALSE if you are no longer

the owner of the credit-card (usually indicating that

the card was removed).

NOTES

This function may be called from within a task, or from a level 1

or level 2 interrupt.

It is highly recommended that you call this function

before accessing credit-card memory, as well as checking

the return value.  If it is a return value of FALSE, you

should stop accessing credit-card memory.

SEE ALSO

OwnCard(), EndCardAccess()

card.resource/CardAccessSpeed                   card.resource/CardAccessSpeed

NAME

CardAccessSpeed -- Select best possible memory access speed.

SYNOPSIS

result=CardAccessSpeed( handle, nanoseconds );

d0			 a1      d0

ULONG CardAccessSpeed( struct CardHandle *, ULONG );

FUNCTION

This function is used to set memory access speed for all CPU

accesses to card memory.

Typically this information would be determined by first examining

the Card Information Structure.

Then you would use this function to let the card.resource

select the best possible access speed for you, however note

that the range of possible access speeds may vary on some

machines (depending on the type of credit-card interface

hardware being provided).

INPUTS

handle - Same handle as that used when OwnCard() was called.

nanoseconds - Preferred access speed in nanoseconds.

RETURNS

Speed - Access speed selected by resource (in nanoseconds).

0  - Not successful.  Either because the credit-card was

removed, or the access speed you requested is slower than

that supported by the credit-card interface hardware.

NOTES

This function may be called from within a task, or from a level 1

or level 2 interrupt.

SEE ALSO

OwnCard()

card.resource/CardChangeCount                   card.resource/CardChangeCount

NAME

CardChangeCount -- Obtain card change count.

SYNOPSIS

count = CardChangeCount( VOID )

d0

ULONG CardChangeChange( VOID );

FUNCTION

This function returns the card change count.  The

counter is incremented by one for every removal, and

for every successful insertion (a card which is inserted

long enough to be debounced before it is removed again).

RESULT

The change count number.

NOTES

This function may be called from a task, or any level interrupt.

SEE ALSO

card.resource/CardForceChange                   card.resource/CardForceChange

NAME

CardForceChange -- Force a card change.

SYNOPSIS

success = CardForceChange( VOID )

d0

BOOL CardForceChange( VOID );

FUNCTION

This function is not intended for general use.  Its

purpose is to force a credit-card change as if

the user had removed, or inserted a card.

This function is intended to be used by a utility program

which needs to force the current card owner to release

ownership of the card, thereby allowing the utility an

opportunity to own the credit-card.

RESULT

TRUE if the function succeeded, FALSE if card change is

not allowed.  This function will generally succeed, unless

someone is using the card in reset remove mode at the time

this function is called.

NOTES

This function should only be called from a task.

SEE ALSO

card.resource/CardInterface                       card.resource/CardInterface

NAME

CardInterface -- Determine the type of card interface.

SYNOPSIS

return = CardInterface()

d0

ULONG CardInterface( void );

FUNCTION

This function is used to determine the type of credit-card

(hardware) interface available.  For the most part the

card.resource hides the hardware details from devices within its

function calls.  However should we need to provide a work-around

because of differences, or limitations imposed by future interface

hardware, this function must be used to identify which interface

is available.

RETURN

A ULONG value as defined in card.h/i.

NOTES

In general only I/O devices (e.g., a device which interfaces

with a modem card) would need to provide work-arounds,

or alternative code.  An example would be a change in the way

interrupt requests from the card are handled.  Specific details

will be provided as need in the future.  I/O devices) should abort

properly if this function returns a value which is unknown.

Current implementations (see card.h/i) -

CARD_INTERFACE_AMIGA_0

-------------------------------------------------------------

The card slot can be configured for use as an I/O interface

by using the CardMiscControl() function.

The card slot inhibits writes to cards which do not negate

the WP status bit.  This can be overridden by using the

CardMiscControl() function.

Changes in the interrupt request line are latched by a gate-array,

and have to be obtained via the status change mechanism provided

when you call the OwnCard() function.  The interrupt is cleared

when you return from the status change interrupt.  A level 2

interrupt is generated.  The interrupt should also be cleared on

the card at this time.  The IRQ line is the same as the RDY/BSY

line.

Changes in BVD1, WP, and RDY/BSY are also latched by the gate-array,

and are obtainable via the status change mechanism provided by

the OwnCard() function.  A level 2 interrupt is generated.

Changes in BVD2 (also used for digital audio) have to be

monitored via polling.  Generally this will cause no problem.

Monitoring changes in BVD1 & BVD2 to monitor for low battery

condition can be handled by a low priority tool which periodically

checks the condition of both lines using the ReadCardStatus()

function.

Some PC oriented eight (8) bit cards may require you read

odd-byte I/O address registers at the corresponding even-byte

address plus 64K.  There is sufficient I/O address space

provided that exceeding I/O address space should not be a problem.

Your code should wait at least 1 millisecond for Vpp to stabilize

after voltage change (see CardProgramVoltage()).

SEE ALSO

CardMiscControl(), resources/card.i, resources/card.h

card.resource/CardMiscControl                   card.resource/CardMiscControl

NAME

CardMiscControl -- Set/Clear miscellaneous control bits

SYNOPSIS

control_bits=CardMiscControl( handle, control_bits );

d0			 	a1      	d1

UBYTE CardMiscControl( struct CardHandle *, UBYTE );

FUNCTION

Used to set/clear miscellaneous control bits (generally for

use with I/O cards).

INPUTS

handle - Same handle as that used when OwnCard() was called.

control_bits - A mask value of control bits to be turned on/off.

The bit values which might be usable are defined in card.h/i.

For example, to enable digital audio, and disable hardware

write-protect (if supported), you would call this function with these

values --

CARDF_DISABLE_WP|CARDF_ENABLE_DIGAUDIO

Then to turn off digital audio, but leave write-protect

disable, you would use a value of --

CARDF_DISABLE_WP

Finally too reenable write protect, call this function with

a mask value of 0.

RETURNS

control_bits - The same mask value you called this function

with if successful.  If one, or more bits has been cleared

in the return mask, this would indicate that the control bit

is not being supported, or that the card has been removed

by the user.

For example, if you called this function with a mask value

of --

CARDF_DISABLE_WP|CARDF_ENABLE_DIGAUDIO

And this function returned a value of --

CARDF_DISABLE_WP

This would indicate that it is not possible to enable digital

audio (most likely because this feature has not been implemented).

NOTES

This function may be called from within a task, or from a level 1

or level 2 interrupt.

!!!IMPORTANT!!!

You should ALWAYS try to enable digital audio for I/O cards

as this will also configure the card socket for the I/O

interface (if supported).

Not all cards will connect the write-enable line (e.g.,

some I/O cards).  On some machines (e.g., the A600) it will

not be possible to write to such cards unless you disable

write-protection by using this function.

SEE ALSO

CardInterface(), resources/card.h, resources/card.i

card.resource/CardProgramVoltage             card.resource/CardProgramVoltage

NAME

CardProgramVoltage -- Set programming voltage.

SYNOPSIS

success=CardProgramVoltage( handle, voltage );

a1      d0

LONG CardProgramVoltage( struct CardHandle *, ULONG );

FUNCTION

Used to set programming voltages (e.g., for FLASH-ROM/EPROM

cards).

INPUTS

handle - Same handle as that used when OwnCard() was called.

voltage - See card.i/h for valid values.

RETURNS

1  - Successful.

0  - Not successful.  Most likely because the credit-card

card has been removed, and you are no longer the owner.

-1 - This function is not being supported.  On some machines

with a minimal (hardware) credit-card interface, this feature

may not be possible.

NOTES

This function may be called from within a task, or from a level 1

or level 2 interrupt.

!!!WARNING!!!

Flash-ROM programming requires careful coding to prevent

leaving the Erase command on too long.  Failure to observe

the maximum time between the Erase command, and the Erase-Verify

command can make a Flash-ROM card unusable.  Some Flash-ROM cards

may provide an internal watch-dog timer which protects the card.

Because of the relatively long time (e.g., 10ms) between Erase, and

Erase-Verify which must be observed, the need for such critical

timing can be problematic on a multi-tasking machine.

Vendors of Flash-ROM's recommend a high priority interrupt

generated by a 10ms timer be used to turn off Erase.  On the

Amiga this can be accomplished by using a CIA-B interval timer.

The timer.device also provides a mechanism for generating a low

priority interrupt.  The timer.device is easier to use than CIA

interval timers, though not as accurate or as safe.

Even if the Flash-ROM card provides an internal watch-dog timer,

implementation of the code during Erase should assume that

the Flash-ROM does not.

SEE ALSO

OwnCard(), resources/card.h, resources/card.i

card.resource/CardResetCard                       card.resource/CardResetCard

NAME

CardResetCard -- Reset credit-card.

SYNOPSIS

success=CardResetCard( handle );

a1

BOOL CardResetCard( struct CardHandle * );

FUNCTION

Used to reset a credit-card.  Some cards, such as some

configurable cards can be reset.

Asserts credit-card reset for at least 10us.

INPUTS

handle - Same handle as that used when OwnCard() was called.

RETURNS

TRUE  - Successful.

FALSE  - Not successful.  Most likely because the credit-card

card has been removed, and you are no longer the owner.

NOTES

This function may be called from within a task, or from a level 1

or level 2 interrupt.

***IMPORTANT***

It is the responsibility of the card owner to reset

configurable cards, or any other type of card such as

some I/O cards before calling ReleaseCard() if the owner

has made use of that card such that it is no longer in its

reset state (unless you are releasing the card because it

has been removed).

If the card manufacturer indicates that a certain amount

of time must elapse between end of reset, and completion

of card initialization, you should wait at least that long

before releasing the card (unless you are releasing the card

because it has been removed).

SEE ALSO

OwnCard(), ReleaseCard()

card.resource/CardResetRemove                   card.resource/CardResetRemove

NAME

CardResetRemove -- Set/Clear reset on card removal.

SYNOPSIS

success=CardResetRemove( handle, flag );

a1      d0

BOOL CardResetRemove( struct CardHandle *, ULONG );

FUNCTION

Used to set/clear HARDWARE RESET on card change detect.

This function should generally not be used by devices

which support HOT-REMOVAL.  HARDWARE RESET on removal

is generally intended for execute-in-place software, or

ram cards whose memory has been added as system ram.

INPUTS

handle - Same handle as that used when OwnCard() was called.

flag - TRUE if you want to SET HARDWARE RESET on credit

card removal.  FALSE if you want to CLEAR HARDWARE

RESET.

RETURNS

1  - Success.

0  - Function failed (most likely because the card was removed

by the user, and you are no longer the owner of the card).

-1 - This function is not being made available.

NOTES

This function should only be called from a task.

SEE ALSO

OwnCard()

card.resource/CopyTuple                               card.resource/CopyTuple

NAME

CopyTuple -- Find/copy a credit card tuple.

SYNOPSIS

success = CopyTuple( CardHandle, buffer, tuplecode, size );

d0			a1	 a0	 d1	    d0

BOOL CopyTuple( struct CardHandle *, UBYTE *, ULONG, ULONG );

FUNCTION

This function scans credit-card memory for a tuple, and

if found, copies it into a supplied buffer.  The entire

tuple (including the tuple code, and link) are copied to

the supplied buffer.  The number of bytes copied to the

buffer will be 2 bytes, plus the argument "size", or the

value in the tuple LINK field (whichever is SMALLER).

The software calling this function is responsible for

examining the copy of the tuple (e.g., recognition of

fields, recognition of stop bytes, etc. within the tuple).

This function does the best job it can to find a tuple

in attribute, or common memory.  It follows tuple chains,

and skips odd bytes in attribute memory.

This function monitors for credit-card removal while reading data.

If the credit-card is removed while a byte is being read, it will

stop searching, and return FALSE.

This function does not protect against another task writing

to credit-card memory while data is being read.  The device

is responsible for single-threading reads/writes to the

credit card as needed.

This function can be used to find multiple tuple codes; this

is a very rare case, so the mechanism provided for doing so is

unusual.  See INPUTS below for more information.

This function does not read bytes within the body of any tuples

except for the tuple you want copied, and the basic compatibility

tuples this function understands (see list below).

On some machines this function may slow down memory access

speed while reading the tuple chain.  This is done to prevent

potential problems on slow cards.  By examining the CISTPL_DEVICE,

and CISTPL_DEVICE_A tuples, you can determine the best possible

memory access speed for the type of card inserted.  Because memory

access speed may be slowed down, calls to this function should

be single-threaded.

The Card Information Structure must start with a CISTPL_DEVICE

tuple stored as the first tuple in attribute memory.  If not,

this function will search for a CISTPL_LINKTARGET tuple

stored at byte 0 of common memory.  Therefore it is possible

to store a CIS on cards which do not have any writeable

attribute memory, though this may cause problems for other

software implemented on other machines.  For example, some

SRAM cards do not come with writeable attribute memory, and/or

some may have OPTIONAL EEPROM memory which may not have been

initialized by the card manufacturer.  While it could be

argued that such cards do not conform to the PCMCIA PC Card

Standard, such cards are cheaper, and therefore likely to be

used.

INPUTS

CardHandle - Same handle as that used when OwnCard() was

called.

buffer - Pointer to a buffer where the tuple will be copied.

The buffer should be at least as large as "size" + 8 (see

use of "size" argument below).  Therefore the minimum buffer

size is 8 bytes.  See NOTES below.

tuplecode - The tuple code you want to search for.  This is

a ULONG value.  The upper 16 bits should be 0, or a number between

1-32K where a value of 0 means you want to find the first tuple

match, a value of 1 the second, etc.  For example -

0x41 means find the FIRST tuple equal to $41.

((1<<16)|(0x41)) means find the SECOND tuple equal to $41.

((2<<16)|(0x41)) means find the THIRD tuple equal to $41.

size - The maximum number of bytes you want copied (not

including the tuple code, and link).  The actual number

of bytes copied may be less than "size" if the tuple

link field is less than "size".

A size of 0 will result in only the tuple code, and

link being copied to your buffer if the tuple is found.

If you do not care how many bytes are copied, any unsigned

value of 255 or greater will do.  In this case a maximum

of 257 bytes might be copied to your buffer (if the

tuple link field is the maximum of 255).

Other sizes are useful if you know the size of the tuple

you want copied, or you know there are active registers

stored within the tuple, and only want to copy a portion

of a tuple.

RETURNS

TRUE if the tuple was found, and copied, else FALSE.

This function may also return false if the CIS is believed

to be corrupt, or if the card is removed while reading the

tuples.

NOTES

This function can be called multiple times (adjusting the "size"

argument) to read a tuple of variable length, or unknown size.

Your supplied buffered is used by this function for working

storage - the contents of it will be modified even if this

function fails to find the tuple you want a copy of.

This function should NOT be used to find/copy tuples of type :

- CISTPL_LONGLINK_A

- CISTPL_LONGLINK_C

- CISTPL_NO_LINK

- CISTPL_LINKTARGET

- CISTPL_NULL

- CISTPL_END

These tuples are automatically handled for you by this function.

SEE ALSO

OwnCard()

card.resource/DeviceTuple                           card.resource/DeviceTuple

NAME

DeviceTuple -- Decode a device tuple

SYNOPSIS

return=DeviceTuple( tuple_data, storage)

a0		a1

ULONG DeviceTuple( UBYTE *, struct DeviceTData *);

FUNCTION

Extracts SIZE, TYPE, and SPEED from a device tuple (generally

obtained with CopyTuple()).

INPUTS

tuple_data - Pointer to a CISTPL_DEVICE tuple (generally obtained

with CopyTuple()).

storage - Pointer to a DeviceTData structure in which results

are to be stored.

struct	DeviceTData {

ULONG	dtd_DTsize;		/* Size of card (bytes)	*/

ULONG	dtd_DTspeed;		/* Speed in nanoseconds	*/

UBYTE	dtd_DTtype;		/* Type of card		*/

UBYTE	dtd_DTflags;		/* Other flags		*/

};

RETURN

SIZE (same as dtd_DTsize) if the Device Tuple could be decoded.

FALSE (0) if the tuple is believed to be invalid.  The tuple is

considered to be invalid if:

The tuple link value is 0.

The device type/speed byte is $00, or $FF.

The device type is DTYPE_EXTEND, which is undefined

as of this writing.

The extended speed byte is a value which is

undefined as of this writing.

The extended speed byte is extended again which is

undefined as of this writing.

The device Size byte is $FF.

NOTES

Some cards may not have a size specified in the device

tuple.  An example would be an I/O card.  The size would be

returned as one (1) in this case.

You should not call this function with a partial CISTPL_DEVICE

tuple, or the return values may be junk.

SEE ALSO

CopyTuple(), resources/card.h, resources/card.i

card.resource/EndCardAccess                       card.resource/EndCardAccess

NAME

EndCardAccess -- Called at the end of credit-card memory access

SYNOPSIS

result=EndCardAccess( handle )

d0			a1

ULONG EndCardAccess( struct CardHandle * );

FUNCTION

This function should be called when you are done accessing

credit-card memory.

Its effect will depend on the type of Amiga machine your

code happens to be running on.  On some machines it

will cause an access light to be turned OFF in approximately

1/2 second.

On machines which support an access light, the light will

automatically be turned off when you call ReleaseCard().

INPUTS

handle - Same handle as that used when OwnCard() was called.

RETURNS

TRUE if you are still the owner of the credit-card.  FALSE

if you are no longer the owner of the credit-card (usually

indicating the card was removed).

NOTES

This function may be called from within a task, or from a level 1

or level 2 interrupt.

It is highly recommended that you call this function

after accessing credit-card memory, as well as checking

the return value.  If it is a return value of FALSE, you

should stop accessing credit-card memory, and conclude

that the card was removed before this function was called.

On some machines it is possible that the credit-card will be

removed before you receive the removed interrupt.

SEE ALSO

OwnCard(), ReleaseCard(), BeginCardAccess()

card.resource/GetCardMap                             card.resource/GetCardMap

NAME

GetCardMap -- Obtain pointer to CardMemoryMap structure

SYNOPSIS

pointer=GetCardMap()

d0

struct CardMemoryMap *GetCardMap( void );

FUNCTION

Obtain pointer to a CardMemoryMap structure.  The structure

is READ only.

Devices should never assume credit-card memory appears

at any particular place in memory.  By using this function

to obtain pointers to the base memory locations of the various

credit-card memory types, your device will continue to work

properly should credit cards appear in different memory

locations in future hardware.

RETURNS

Pointer to CardMemoryMap structure -

struct CardMemoryMap {

UBYTE	*cmm_CommonMemory;

UBYTE	*cmm_AttributeMemory;

UBYTE	*cmm_IOMemory;

};

NOTES

If any pointer in the structure is NULL, it means this type

of credit-card memory is not being made available.

SEE ALSO

resources/card.h, resources/card.i

card.resource/IfAmigaXIP                             card.resource/IfAmigaXIP

NAME

IfAmigaXIP -- Check if a card is an Amiga execute-in-place card.

SYNOPSIS

result=IfAmigaXIP( handle )

d0		       a2

struct Resident *IfAmigaXIP( struct CardHandle * );

FUNCTION

Check to see if a card in the slot is an Amiga execute-in-place

card.  The Card Information Structure must have a valid

CISTPL_AMIGAXIP tuple.

Tuples can be treated like structures.  The format of a

CISTPL_AMIGAXIP tuple is -

struct TP_AmigaXIP {

UBYTE	TPL_CODE;

UBYTE	TPL_LINK;

UBYTE	TP_XIPLOC[4];

UBYTE	TP_XIPFLAGS;

UBYTE	TP_XIPRESRV;

};

The TPL_CODE field must be equal to CISTPL_AMIGAXIP (0x91).

The TPL_LINK field must be equal to the length of the body

of a CISTPL_AMIGAXIP tuple (0x06).

The TP_XIPLOC array is the memory location of your ROM-TAG

stored in little-endian order.  This value is stored as an

"offset" into common memory as is the standard for storing 32 bit

bit pointers in tuples.

For example, a pointer to a ROM-TAG stored at an offset of

0x00000200 would be stored as four bytes like so:

0x00, 0x02, 0x00, 0x00

Currently credit-card common memory is mapped starting at

memory location 0x600000.  Because a ROM-TAG is used,

it is implied that execute-in-place code can be compiled/linked

to use absolute references.  It is believed that most developers

will not want to have to write pc-relative code only.

The TP_XIPFLAGS field is treated as a set of flag bits.

See card.i/h for defined bits.  All undefined bits MUST be

0.

The TP_XIPRESRV field must be 0 for now.

The system polls for cards which have a CISTPL_AMIGAXIP tuple

at the same time that it searches for devices to boot off.

When a card with a valid CISTPL_AMIGAXIP tuple is found, the

system will call your ROM-TAG via Exec's InitResident() function.

The system examines the return code from InitResident().  A

NULL return in D0 means you are done with the card, and it can

be removed by the user.  A successful return indicates you are

still using the card.  Some programs (e.g., some games) may

never return.  The only requirement is that if you do return,

you must leave the system in a "good" state (including returning

most of, or all the memory you allocated).  The standard

convention for preserving registers apply.

Note that your execute-in-place code will not be called

a second time, unless you have returned a non-successful

result.  In this case your execute-in-place code MUST

assume the user can remove, and insert your card again.

There are a variety of ways to check for re-insertion

(e.g., search for a message port, device, library, task,

etc., that you created).

Note that your execute-in-place code runs in an environment

similar to boot block games; before DOS has been initialized!

Your execute-in-place code should NOT try to initialize DOS

because DOS requires a suitable disk-like device be at

the head of the expansion base mountlist to boot off.

If you need DOS, it is possible to boot off a credit-card using

carddisk.device.  Such cards will require a valid

CISTPL_DEVICE tuple, and CISTPL_FORMAT tuple.  A portion

of the card can be used for a minimal number of data blocks

like the method described above.

However this method is not recommended, though it is anticipated

that some developers will have thought of, and used this

method anyway.  If you must do this, at least use the

CardHandle returned by OwnCard() to set hardware reset

on removal else the machine will likely crash anyway if

the card is removed while your execute-in-place code is running.

RETURNS

Pointer to a ROM-TAG on the card, or NULL indicating that:

o The card does not meet the above requirements, or

o The card has been removed, or

o You are not the owner of the credit-card.

NOTES

This function is being made public so developers can test

test that their execute-in-place credit-cards meet the

requirements for an Amiga execute-in-place card.  In general

there is no reason for devices, or applications to use this

function.

Amiga execute-in-place software is identified via a tuple code

reserved for manufacturer specific use, therefore the manufacturer

of the card may freely use the CISTPL_VERS_1, or CISTPL_VERS_2

tuples to place identification information on the credit-card.

No attempt has been made to make use of the MS-DOS execute-in-place

method; it is believed that most manufacturers of Amiga

execute-in-place software will prefer a simple, and small

scheme for running their execute-in-place code.

SEE ALSO

OwnCard(), resources/card.h, resources/card.i

card.resource/OwnCard                                   card.resource/OwnCard

NAME

OwnCard -- Own credit card registers, and memory

SYNOPSIS

return = OwnCard( handle )

d0		a1

struct CardHandle *OwnCard( struct CardHandle * );

FUNCTION

This function is used to obtain immediate, or deferred

ownership of a credit-card in the credit-card slot.

Typically an EXEC STYLE DEVICE will be written to interface

between an application, and a credit card in the slot.  While

applications, and libraries can attempt to own a credit-card

in the card slot, the rest of this documentation assumes a

device interface will be used.

Because credit-cards can be inserted, or removed by the user at

any time (otherwise known as HOT-INSERTION, and HOT-REMOVAL),

the card.resource provides devices with a protocol which

lets many devices bid for ownership of a newly inserted card.

In general, devices should support HOT-REMOVAL, however there

are legitimate cases where HOT-REMOVAL is not practical.  For

these cases this function allows you to own the resource using

the CARDB_RESETREMOVE flag.  If the card is removed before your

device calls ReleaseCard(), the machine will RESET.

INPUTS

handle - pointer to a CardHandle structure.

struct CardHandle {

struct Node 		 cah_CardNode;

struct Interrupt	*cah_CardRemoved;

struct Interrupt	*cah_CardInserted;

struct Interrupt	*cah_CardStatus;

UBYTE			 cah_CardFlags;

};

The following fields in the structure must be filled

in by the application before calling OwnCard() -

cah_CardNode.ln_Pri -

See table below.  The Node field is used by the resource to

add your handle to a sorted list of CardHandle structures.

This list is used by the resource to notify devices when the

device owns the credit-card.

Your device will only be notified (at most) one time

per card insertion, and perhaps less often if some

higher priority device on the notification list retains

ownership of a card in the slot.

Priority	Comments

--------------------------------------------------

>= 21	Reserved for future use

10-20	To be used by third party devices (e.g.,

I/O CARD manufacturers) which look for

specific card tuples to identify credit-cards.

01-19	Reserved for future use

00	To be used by general purpose devices which

have loose card specification requirements.

<= -1	Reserved for future use

cah_CardNode.ln_Type -

Must be set to 0 for now.  This field may be used in the

future to identify an extended CardHandle structure.

cah_CardNode.ln_Name -

Must be initialized to NULL, or name of device which owns

this structure.

cah_CardRemoved -

Pointer to an initialized interrupt structure.  Only the

is_Data, and is_Code fields need to be initialized.  This

is the interrupt code which will be called when a credit-card

which your device owns is removed.  Once you receive this

interrupt, all credit-card interface control registers are

reset (e.g., programming voltage, access speed, etc.), and

you should stop accessing the card as soon as possible.

Because your code is called on interrupt time, you

should do the least amount possible, and use little

stack space.

This pointer can be NULL if you have asked for reset

on card-removal, and you never turn reset off.

cah_ CardInserted -

Pointer to an initialized interrupt structure.  Only the

is_Data, and is_Code fields need to be initialized.  This

is the code which will be called when your CardHandle owns

the credit-card in the slot.

Note that your code may be called on the context of

an interrupt, or a task in FORBID, therefore you should

do the least amount possible, and use little stack space.

Note that it is possible to receive a card removed

interrupt immediately before you receive this interrupt if

the card is removed while your CardInserted interrupt

is being called.

Your device owns the credit-card until the card is manually

removed by the user, or you release the card by calling

ReleaseCard().

Your device should examine the card in the slot (e.g., look

for specific tuples), and decide if the card is of a type your

device understands.

If not, release ownership of the card by calling

ReleaseCard() so that other devices will be given a chance to

examine the current card in the credit-card slot.

cah_CardStatus -

Pointer to an initialized interrupt structure.  Only the

is_Data, and is_Code fields need to be initialized.

Note that your code will be called on the context of

an interrupt, therefore you should do the least

amount possible, and use little stack space.

Note that it is possible to receive a card removed

interrupt immediately before you receive this interrupt

if the card is removed during this interrupt.

If this pointer is NULL, you will not receive card status

change interrupts.

Your interrupt code will be called with a mask value in

register D0, and a pointer to your data in A1.

The mask value in D0 can be interpreted using the same bit

definitions returned by ReadCardStatus().  Note that more

than one bit may be set, and the mask only tells you what has

changed, not the current state.

Use ReadCardStatus() if you need to determine the current

state of the status bits.

Not all status change interrupts will necessarily be

enabled on all systems.  For example, on some systems

BVD2/DA status change interrupts will not be enabled so

that digital audio can occur without generating many

interrupts.  Status change interrupts are typically meant

to be used for monitoring BSY/IRQ, WR, and BVD1/SC.  Battery

voltage low detection would best be done by a separate

utility which periodically polls BVD1 & BVD2 by using the

ReadCardStatus() function.

Typically the mask value in D0 MUST be returned unchanged

on exit from your code.  The return value in D0 is then used

to clear the source(s) of the interrupt.

In the rare case that you need to keep a status change

interrupt active, clear the appropriate bit(s) in D0 before

returning via RTS.  Clear no bits other than those defined

as valid bits for ReadCardStatus()!

cah_CardFlags -

Optional flags (all other bits must be 0).

- CARDB_RESETREMOVE means you want the machine to

reset if the credit-card in the slot is removed

while you own the credit-card.

- CARDB_IFAVAILABLE means you only want ownership of

the credit-card in the slot if it is immediately

available.

If it is available, your CardHandle structure will be added

to a list so that you can be notified via an interrupt when

the credit-card is removed by the user.

If the credit-card is not immediately available (either

because there is no credit-card in the slot, or because some

other device owns the credit-card), your CardHandle structure

will NOT be added to the notification list.

- CARDB_DELAYOWNERSHIP means you never want a successful

return from OwnCard() even if the credit-card is

available.  Rather you will be notified of ownership

via your cah_CardInserted interrupt.  If you use this flag,

OwnCard() will always return -1.  This flag cannot be used

with the CARDB_IFAVAILABLE flag.

ALL other fields are used by the resource, and no fields in the

structure may be modified while the structure is in use by the

resource.  If you need to make changes, you must remove your

CardHandle (see ReleaseCard()), make the changes, and then call

OwnCard() again.

RESULTS

0  - indicates success, your device owns the credit card.

-1  - indicates that the card cannot be owned (most likely

because there is no card in the credit card slot).

ptr - indicates failure.  Returns pointer to the CardHandle

structure which owns the credit card.

NOTES

This function should only be called from a task.

CardHandle interrupts are called with a pointer to your data

in A1, and a pointer to your code in A5.  With the exception

of status change interrupts, D0-D1, A0-A1, and A5-A6 may be

treated as scratch registers.  Status change interrupts are

also called with meaningful data in D0, and expect D0 be

preserved upon RTS from your code.  No other registers are

guaranteed to contain initialized data.  All other registers

must be preserved.

SEE ALSO

ReleaseCard(), ReadCardStatus(), resources/card.i, resources/card.h

card.resource/ReadCardStatus                     card.resource/ReadCardStatus

NAME

ReadCardStatus -- Read credit card status register

SYNOPSIS

status=ReadCardStatus()

d0

UBYTE ReadCardStatus( void );

FUNCTION

Returns current state of the credit card status register.

See card.h/i for bit definitions.

Note that the meaning of the returned status bits may vary

depending on the type of card inserted in the slot, and

mode of operation.  Interpretation of the bits is left

up to the application.

RETURNS

A UBYTE value to be interpreted as status bits.

NOTES

This function may be called from within a task, or from any level

interrupt.

SEE ALSO

resources/card.h, resources/card.i

card.resource/ReleaseCard                           card.resource/ReleaseCard

NAME

ReleaseCard -- Release ownership of credit card

SYNOPSIS

ReleaseCard( handle, flags )

a1	     d0

void ReleaseCard( struct CardHandle *, ULONG );

FUNCTION

This function releases ownership of the credit card in the

slot.

The access light (if any) is automatically turned off

(if it was turned on) when you release ownership of

a card you owned, and all credit-card control registers

are reset to their default state.

You must call this function if -

You own the credit-card, and want to release it so that

other devices on the notification list will have a chance

to examine the credit-card in the card slot.

You took a Card Removed interrupt while you owned the

credit-card.  If so, you MUST call this function, else

no other task will be notified of newly inserted cards.  On

some machines the credit-card interface hardware may also

be left disabled until you respond to the card removed interrupt

by calling this function.

You want to remove yourself from the notification list (see

optional flags below).

INPUTS

handle - Same handle as that used when OwnCard() was called.

flags - Optional flags.

- CARDB_REMOVEHANDLE means you want remove your

CardHandle structure from the notification list

whether or not you currently own the credit-card

in the card slot.  The node structure in your

CardHandle will be removed from the notification

list, and ownership will be released if you were

the owner of the card.

NOTES

This function should only be called from a task.

SEE ALSO

OwnCard(), resources/card.i, resources/card.h