TABLE OF CONTENTS

audio.device/AbortIO

audio.device/BeginIO

audio.device/CloseDevice

audio.device/command/ADCMD_ALLOCATE

audio.device/command/ADCMD_FINISH

audio.device/command/ADCMD_FREE

audio.device/command/ADCMD_LOCK

audio.device/command/ADCMD_PERVOL

audio.device/command/ADCMD_SETPREC

audio.device/command/ADCMD_WAITCYCLE

audio.device/command/CMD_CLEAR

audio.device/command/CMD_FLUSH

audio.device/command/CMD_READ

audio.device/command/CMD_RESET

audio.device/command/CMD_START

audio.device/command/CMD_STOP

audio.device/command/CMD_UPDATE

audio.device/command/CMD_WRITE

audio.device/Expunge

audio.device/OpenDevice

audio.device/AbortIO                                       audio.device/AbortIO

NAME

AbortIO - abort a device command

SYNOPSIS

AbortIO(iORequest);

A1

FUNCTION

AbortIO tries to abort a device command.  It is allowed to be

unsuccessful.  If the Abort is successful, the io_Error field of the

iORequest contains an indication that IO was aborted.

INPUTS

iORequest -- pointer to the I/O Request for the command to abort

audio.device/BeginIO                                       audio.device/BeginIO

NAME

BeginIO - dispatch a device command

SYNOPSIS

BeginIO(iORequest);

A1

FUNCTION

BeginIO has the responsibility of dispatching all device commands.

Immediate commands are always called directly, and all other commands

are queued to make them single threaded.

INPUTS

iORequest -- pointer to the I/O Request for this command

audio.device/CloseDevice                               audio.device/CloseDevice

NAME

CloseDevice - terminate access to the audio device

SYNOPSIS

CloseDevice(iORequest);

A1

FUNCTION

The CloseDevice routine notifies the audio device that it will no

longer be used.  It takes an I/O audio request block (IOAudio) and

clears the device pointer (io_Device).  If there are any channels

allocated with the same allocation key (ioa_AllocKey), CloseDevice

frees (ADCMD_FREE) them. CloseDevice decrements the open count, and if

it falls to zero and an expunge (Expunge) is pending, the device is

expunged.

INPUTS

iORequest   - pointer to audio request block (struct IOAudio)

io_Device   - pointer to device node, must be set by (or

copied from I/O block set by) open (OpenDevice)

io_Unit     - bit map of channels to free (ADCMD_FREE) (bits 0

thru 3 correspond to channels 0 thru 3)

ioa_AllocKey- allocation key, used to free channels

OUTPUTS

iORequest - pointer to audio request block (struct IOAudio)

io_Device   - set to -1

io_Unit     - set to zero

audio.device/command/ADCMD_ALLOCATE

NAME

ADCMD_ALLOCATE -- allocate a set of audio channels

FUNCTION

ADCMD_ALLOCATE is a command that allocates multiple audio channels.

ADCMD_ALLOCATE takes an array of possible channel combinations

(ioa_Data) and an allocation precedence (ln_Pri) and tries to allocate

one of the combinations of channels.

If the channel combination array is zero length (ioa_Length), the

allocation succeeds; otherwise, ADCMD_ALLOCATE checks each

combination, one at a time, in the specified order, to find one

combination that does not require ADCMD_ALLOCATE to steal allocated

channels.

If it must steal allocated channels, it uses the channel combination

that steals the lowest precedence channels.

ADCMD_ALLOCATE cannot steal a channel of equal or greater precedence

than the allocation precedence (ln_Pri).

If it fails to allocate any channel combination and the no-wait flag

(ADIOF_NOWAIT) is set ADCMD_ALLOCATE returns a zero in the unit field

of the I/O request (io_Unit) and an error (IOERR_ALLOCFAILED).  If the

no-wait flag is clear, it places the I/O request in a list that tries

to allocate again whenever ADCMD_FREE frees channels or ADCMD_SETPREC

lowers the channels' precedences.

If the allocation is successful, ADCMD_ALLOCATE checks if any channels

are locked (ADCMD_LOCK) and if so, replies (ReplyMsg) the lock I/O

request with an error (ADIOERR_CHANNELSTOLEN). Then it places the

allocation I/O request in a list waiting for the locked channels to be

freed.  When all the allocated channels are un-locked, ADCMD_ALLOCATE:

. resets (CMD_RESET) the allocated channels,

. generates a new allocation key (ioa_AllocKey), if it is zero,

. copies the allocation key into each of the allocated channels

. copies the allocation precedence into each of the allocated

channels, and

. copies the channel bit map into the unit field of the I/O request.

If channels are allocated with a non-zero allocation key,

ADCMD_ALLOCATE allocates with that same key; otherwise, it generates a

new and unique key.

ADCMD_ALLOCATE is synchronous:

. if the allocation succeeds and there are no locked channels to be

stolen, or

. if the allocation fails and the no-wait flag is set.

. if the allocation fails and the no-wait flag is set.

In either case, ADCMD_ALLOCATE only replies (mn_ReplyPort) if the

quick flag (IOF_QUICK) is clear; otherwise, the allocation is

asynchronous, so it clears the quick flag and replies the I/O request

after the allocation is finished.  If channels are stolen, all audio

device commands return an error (IOERR_NOALLOCATION) when the former

user tries to use them again.  Do not use ADCMD_ALLOCATE in interrupt

code.

If you decide to store directly to the audio hardware registers, you

must either lock the channels you've allocated, or set the precedence

to maximum (ADALLOC_MAXPREC) to prevent the channels from being

stolen.

Under all circumstances, unless channels are stolen, you must free

(ADCMD_FREE) all allocated channels when you are finished using them.

INPUTS

ln_Pri      - allocation precedence (-128 thru 127)

mn_ReplyPort- pointer to message port that receives I/O request after

the allocation completes is asynchronous or quick flag

(ADIOF_QUICK) is set

io_Device   - pointer to device node, must be set by (or copied from

I/O block set by) OpenDevice function

io_Command  - command number for ADCMD_ALLOCATE

io_Flags    - flags, must be cleared if not used:

IOF_QUICK   - (CLEAR) reply I/O request

(SET) only reply I/O request only if

asynchronous (see above text)

ADIOF_NOWAIT- (CLEAR) if allocation fails, wait till is

succeeds

(SET) if allocation fails, return error

(ADIOERR_ALLOCFAILED)

ioa_AllocKey- allocation key, zero to generate new key; otherwise,

it must be set by (or copied from I/O block set by)

OpenDevice function or previous ADCMD_ALLOCATE command

ioa_Data    - pointer to channel combination options (byte array, bits

0 thru 3 correspond to channels 0 thru 3)

ioa_Length  - length of the channel combination option array

(0 thru 16, 0 always succeeds)

OUTPUTS

io_Unit     - bit map of successfully allocated channels (bits 0 thru

3 correspond to channels 0 thru 3)

io_Flags    - IOF_QUICK flag cleared if asynchronous (see above text)

io_Error    - error number:

0                   - no error

ADIOERR_ALLOCFAILED - allocation failed

ioa_AllocKey- allocation key, set to a unique number if passed a zero

and command succeeds

audio.device/command/ADCMD_FINISH             audio.device/command/ADCMD_FINISH

NAME

ADCMD_FINISH -- abort writes in progress to audio channels

FUNCTION

ADCMD_FINISH is a command for multiple audio channels.  For each

selected channel (io_Unit), if the allocation key (ioa_AllocKey) is

correct and there is a write (CMD_WRITE)in progress, ADCMD_FINISH

aborts the current write immediately or at the end of the current

cycle depending on the sync flag (ADIOF_SYNCCYCLE).  If the allocation

key is incorrect ADCMD_FINISH returns an error (ADIOERR_NOALLOCATION).

ADCMD_FINISH is synchronous and only replies (mn_ReplyPort) if the

quick flag (IOF_QUICK) is clear.  Do not use ADCMD_FINISH in interrupt

code at interrupt level 5 or higher.

INPUTS

mn_ReplyPort- pointer to message port that receives I/O request

if the quick flag (IOF_QUICK) is clear

io_Device   - pointer to device node, must be set by (or copied from

I/O block set by) OpenDevice function

io_Unit     - bit map of channels to finish (bits 0 thru 3 correspond

to channels 0 thru 3)

io_Command  - command number for ADCMD_FINISH

io_Flags    - flags, must be cleared if not used:

IOF_QUICK      - (CLEAR) reply I/O request

ADIOF_SYNCCYCLE- (CLEAR) finish immediately

(SET) finish at the end of current

cycle

ioa_AllocKey- allocation key, must be set by (or copied from I/O block

set by) OpenDevice function or ADCMD_ALLOCATE command

OUTPUTS

io_Unit     - bit map of channels successfully finished (bits 0 thru 3

correspond to channels 0 thru 3)

io_Error    - error number:

0                    - no error

ADIOERR_NOALLOCATION - allocation key (ioa_AllocKey)

does not match key for channel

audio.device/command/ADCMD_FREE                 audio.device/command/ADCMD_FREE

NAME

ADCMD_FREE -- free audio channels for allocation

FUNCTION

ADCMD_FREE is a command for multiple audio channels.  For each

selected channel (io_Unit), if the allocation key (ioa_AllocKey) is

correct, ADCMD_FREE does the following:

. restores the channel to a known state (CMD_RESET),

. changes the channels allocation key, and

. makes the channel available for re-allocation.

. If the channel is locked (ADCMD_LOCK) ADCMD_FREE unlocks it and

clears the bit for the channel (io_Unit) in the lock I/O request.

If the lock I/O request has no channel bits set ADCMD_FREE replies

the lock I/O request, and

. checks if there are allocation requests (ADCMD_ALLOCATE) waiting

for the channel.

Otherwise, ADCMD_FREE returns an error (ADIOERR_NOALLOCATION).

ADCMD_FREE is synchronous and only replies (mn_ReplyPort) if the quick

flag (IOF_QUICK) is clear.  Do not use ADCMD_FREE in interrupt code.

INPUTS

mn_ReplyPort- pointer to message port that receives I/O request

if the quick flag (IOF_QUICK) is clear

io_Device   - pointer to device node, must be set by (or copied from

I/O block set by) OpenDevice function

io_Unit     - bit map of channels to free (bits 0 thru 3 correspond to

channels 0 thru 3)

io_Command  - command number for ADCMD_FREE

io_Flags    - flags, must be cleared if not used:

IOF_QUICK - (CLEAR) reply I/O request

ioa_AllocKey- allocation key, must be set by (or copied from I/O block

set by) OpenDevice function or ADCMD_ALLOCATE command

OUTPUTS

io_Unit     - bit map of channels successfully freed (bits 0 thru 3

correspond to channels 0 thru 3)

io_Error    - error number:

0                    - no error

ADIOERR_NOALLOCATION - allocation key (ioa_AllocKey)

does not match key for channel

audio.device/command/ADCMD_LOCK                 audio.device/command/ADCMD_LOCK

NAME

ADCMD_LOCK -- prevent audio channels from being stolen

FUNCTION

ADCMD_LOCK is a command for multiple audio channels.  For each

selected channel (io_Unit), if the allocation key (ioa_AllocKey) is

correct, ADCMD_LOCK locks the channel, preventing subsequent

allocations (ADCMD_ALLOCATE or OpenDevice) from stealing the channel.

Otherwise, ADCMD_LOCK returns an error (ADIOERR_NOALLOCATION) and will

not lock any channels.

Unlike setting the precedence (ADCMD_SETPREC, ADCMD_ALLOCATE or

OpenDevice) to maximum (ADALLOC_MAXPREC) which would cause all

subsequent allocations to fail, ADCMD_LOCK causes all higher

precedence allocations, even no-wait (ADIOF_NOWAIT) allocations, to

wait until the channels are un-locked.

Locked channels can only be unlocked by freeing them (ADCMD_FREE),

which clears the channel select bits (io_Unit).  ADCMD_LOCK does not

reply the I/O request (mn_ReplyPort) until all the channels it locks

are freed, unless a higher precedence allocation attempts to steal one

the locked channels. If a steal occurs, ADCMD_LOCK replies and returns

an error (ADIOERR_CHANNELSTOLEN).  If the lock is replied

(mn_ReplyPort) with this error, the channels should be freed as soon

as possible.  To avoid a possible deadlock, never make the freeing of

stolen channels dependent on another allocations completion.

ADCMD_LOCK is only asynchronous if the allocation key is correct, in

which case it clears the quick flag (IOF_QUICK); otherwise, it is

synchronous and only replies if the quick flag (IOF_QUICK) is clear.

Do not use ADCMD_LOCK in interrupt code.

INPUTS

mn_ReplyPort- pointer to message port that receives I/O request

if the quick flag (IOF_QUICK) is clear

io_Device   - pointer to device node, must be set by (or copied from

I/O block set by) OpenDevice function

io_Unit     - bit map of channels to lock (bits 0 thru 3 correspond to

channels 0 thru 3)

io_Command  - command number for ADCMD_LOCK

io_Flags    - flags, must be cleared

ioa_AllocKey- allocation key, must be set by (or copied from I/O block

set by) OpenDevice function or ADCMD_ALLOCATE command

OUTPUTS

io_Unit     - bit map of successfully locked channels (bits 0 thru 3

correspond to channels 0 thru 3) not freed (ADCMD_FREE)

io_Flags    - IOF_QUICK flag cleared if the allocation key is correct

(no ADIOERR_NOALLOCATION error)

io_Error    - error number:

0                    - no error

ADIOERR_NOALLOCATION - allocation key (ioa_AllocKey)

does not match key for channel

ADIOERR_CHANNELSTOLEN- allocation attempting to steal

locked channel

audio.device/command/ADCMD_PERVOL             audio.device/command/ADCMD_PERVOL

NAME

ADCMD_PERVOL -- change the period and volume for writes in progress to

audio channels

FUNCTION

ADCMD_PERVOL is a command for multiple audio channels.  For each

selected channel (io_Unit), if the allocation key (ioa_AllocKey) is

correct and there is a write (CMD_WRITE) in progress, ADCMD_PERVOL

loads a new volume and period immediately or at the end of the current

cycle depending on the sync flag (ADIOF_SYNCCYCLE).  If the allocation

key in incorrect, ADCMD_PERVOL returns an error

(ADIOERR_NOALLOCATION).  ADCMD_PERVOL is synchronous and only replies

(mn_ReplyPort) if the quick flag (IOF_QUICK) is clear.  Do not use

ADCMD_PERVOL in interrupt code at interrupt level 5 or higher.

INPUTS

mn_ReplyPort- pointer to message port that receives I/O request

if the quick flag (IOF_QUICK) is clear

io_Device   - pointer to device node, must be set by (or copied from

I/O block set by) OpenDevice function

io_Unit     - bit map of channels to load period and volume (bits 0

thru 3 correspond to channels 0 thru 3)

io_Command  - command number for ADCMD_PERVOL

io_Flags    - flags, must be cleared if not used:

IOF_QUICK      - (CLEAR) reply I/O request

ADIOF_SYNCCYCLE- (CLEAR) load period and volume

immediately

(SET) load period and volume at the end

of the current cycle

ioa_AllocKey- allocation key, must be set by (or copied from I/O block

set by) OpenDevice function or ADCMD_ALLOCATE command

ioa_Period  - new sample period in 279.365 ns increments (124 thru

65536, anti-aliasing filter works below 300 to 500

depending on waveform)

ioa_Volume  - new volume (0 thru 64, linear)

OUTPUTS

io_Unit     - bit map of channels that successfully loaded period and

volume (bits 0 thru 3 correspond to channels 0 thru 3)

io_Error    - error number:

0                    - no error

ADIOERR_NOALLOCATION - allocation key (ioa_AllocKey)

does not match key for channel

audio.device/command/ADCMD_SETPREC           audio.device/command/ADCMD_SETPREC

NAME

ADCMD_SETPREC -- set the allocation precedence for audio channels

FUNCTION

ADCMD_SETPREC is a command for multiple audio channels.  For each

selected channel (io_Unit), if the allocation key (ioa_AllocKey) is

correct, ADCMD_SETPREC sets the allocation precedence to a new value

(ln_Pri) and checks if there are allocation requests (ADCMD_ALLOCATE)

waiting for the channel which now have higher precedence; otherwise,

ADCMD_SETPREC returns an error (ADIOERR_NOALLOCATION).  ADCMD_SETPREC

is synchronous and only replies (mn_ReplyPort) if the quick flag

(IOF_QUICK) is clear.  Do not use ADCMD_SETPREC in interrupt code.

INPUTS

ln_Pri      - new allocation precedence (-128 thru 127)

mn_ReplyPort- pointer to message port that receives I/O request

if the quick flag (IOF_QUICK) is clear

io_Device   - pointer to device node, must be set by (or copied from

I/O block set by) OpenDevice function

io_Unit     - bit map of channels to set precedence (bits 0 thru 3

correspond to channels 0 thru 3)

io_Command  - command number for ADCMD_SETPREC

io_Flags    - flags, must be cleared if not used:

IOF_QUICK - (CLEAR) reply I/O request

ioa_AllocKey- allocation key, must be set by (or copied from I/O block

set by) OpenDevice function or ADCMD_ALLOCATE command

OUTPUTS

io_Unit     - bit map of channels that successfully set precedence

(bits 0 thru 3 correspond to channels 0 thru 3)

io_Error    - error number:

0                    - no error

ADIOERR_NOALLOCATION - allocation key (ioa_AllocKey)

does not match key for channel

audio.device/command/ADCMD_WAITCYCLE

NAME

ADCMD_WAITCYCLE -- wait for an audio channel to complete the current

cycle of a write

FUNCTION

ADCMD_WAITCYCLE is a command for a single audio channel (io_Unit).

If the allocation key (ioa_AllocKey) is correct and there is a write

(CMD_WRITE) in progress on selected channel, ADCMD_WAITCYCLE does not

reply (mn_ReplyPort) until the end of the current cycle.  If there is

no write is progress, ADCMD_WAITCYCLE replies immediately.  If the

allocation key is incorrect, ADCMD_WAITCYCLE returns an error

(ADIOERR_NOALLOCATION).  ADCMD_WAITCYCLE returns an error

(IOERR_ABORTED) if it is canceled (AbortIO) or the channel is stolen

(ADCMD_ALLOCATE).  ADCMD_WAITCYCLE is only asynchronous if it is

waiting for a cycle to complete, in which case it clears the quick

flag (IOF_QUICK); otherwise, it is synchronous and only replies if the

quick flag (IOF_QUICK) is clear.  Do not use ADCMD_WAITCYCLE in

interrupt code at interrupt level 5 or higher.

INPUTS

mn_ReplyPort- pointer to message port that receives I/O request, if

the quick flag (IOF_QUICK) is clear, or if a write is in

progress on the selected channel and a cycle has

completed

io_Device   - pointer to device node, must be set by (or copied from

I/O block set by) OpenDevice function

io_Unit     - bit map of channel to wait for cycle (bits 0 thru 3

correspond to channels 0 thru 3), if more then one bit

is set lowest bit number channel is used

io_Command  - command number for CMD_WAITCYCLE

io_Flags    - flags, must be cleared if not used:

IOF_QUICK - (CLEAR) reply I/O request

(SET) only reply I/O request if a write is

in progress on the selected channel

and a cycle has completed

ioa_AllocKey- allocation key, must be set by (or copied from I/O block

set by) OpenDevice function or ADCMD_ALLOCATE command

OUTPUTS

io_Unit     - bit map of channel that successfully waited for cycle

(bits 0 thru 3 correspond to channels 0 thru 3)

io_Flags    - IOF_QUICK flag cleared if a write is in progress on the

selected channel

io_Error    - error number:

0                    - no error

IOERR_ABORTED        - canceled (AbortIO) or channel

stolen

ADIOERR_NOALLOCATION - allocation key (ioa_AllocKey)

does not match key for channel

audio.device/command/CMD_CLEAR                   audio.device/command/CMD_CLEAR

NAME

CMD_CLEAR -- throw away internal caches

FUNCTION

CMD_CLEAR is a standard command for multiple audio channels.  For each

selected channel (io_Unit), if the allocation key (ioa_AllocKey) is

correct, CMD_CLEAR does nothing; otherwise, CMD_CLEAR returns an error

(ADIOERR_NOALLOCATION).  CMD_CLEAR is synchronous and only replies

(mn_ReplyPort) if the quick flag (IOF_QUICK) is clear.

INPUTS

mn_ReplyPort- pointer to message port that receives I/O request after

if the quick flag (IOF_QUICK) is clear

io_Device   - pointer to device node, must be set by (or copied from

I/O block set by) OpenDevice function

io_Unit     - bit map of channels to clear (bits 0 thru 3 correspond

to channels 0 thru 3)

io_Command  - command number for CMD_CLEAR

io_Flags    - flags, must be cleared if not used:

IOF_QUICK - (CLEAR) reply I/O request

ioa_AllocKey- allocation key, must be set by (or copied from I/O block

set by) OpenDevice function or ADCMD_ALLOCATE command

OUTPUTS

io_Unit     - bit map of channels successfully cleared (bits 0 thru 3

correspond to channels 0 thru 3)

io_Error    - error number:

0                    - no error

ADIOERR_NOALLOCATION - allocation key (ioa_AllocKey)

does not match key for channel

audio.device/command/CMD_FLUSH                   audio.device/command/CMD_FLUSH

NAME

CMD_FLUSH -- cancel all pending I/O

FUNCTION

CMD_FLUSH is a standard command for multiple audio channels.  For each

selected channel (io_Unit), if the allocation key (ioa_AllocKey) is

correct, CMD_FLUSH aborts all writes (CMD_WRITE) in progress or queued

and any I/O requests waiting to synchronize with the end of the cycle

(ADCMD_WAITCYCLE); otherwise, CMD_FLUSH returns an error

(ADIOERR_NOALLOCATION).  CMD_FLUSH is synchronous and only replies

(mn_ReplyPort) if the quick flag (IOF_QUICK) is clear.  Do not use

CMD_FLUSH in interrupt code at interrupt level 5 or higher.

INPUTS

mn_ReplyPort- pointer to message port that receives I/O request

if the quick flag (IOF_QUICK) is clear

io_Device   - pointer to device node, must be set by (or copied from

I/O block set by) OpenDevice function

io_Unit     - bit map of channels to flush (bits 0 thru 3 correspond

to channels 0 thru 3)

io_Command  - command number for CMD_FLUSH

io_Flags    - flags, must be cleared if not used:

IOF_QUICK - (CLEAR) reply I/O request

ioa_AllocKey- allocation key, must be set by (or copied from I/O block

set by) OpenDevice function or ADCMD_ALLOCATE command

OUTPUTS

io_Unit     - bit map of channels successfully flushed (bits 0 thru 3

correspond to channels 0 thru 3)

io_Error    - error number:

0                    - no error

ADIOERR_NOALLOCATION - allocation key (ioa_AllocKey)

does not match key for channel

audio.device/command/CMD_READ                     audio.device/command/CMD_READ

NAME

CMD_READ -- normal I/O entry point

FUNCTION

CMD_READ is a standard command for a single audio channel (io_Unit).

If the allocation key (ioa_AllocKey) is correct, CMD_READ returns a

pointer (io_Data) to the I/O block currently writing (CMD_WRITE) on

the selected channel; otherwise, CMD_READ returns an error

(ADIOERR_NOALLOCATION).  If there is no write in progress, CMD_READ

returns zero.  CMD_READ is synchronous and only replies (mn_ReplyPort)

if the quick bit (IOF_QUICK) is clear.

INPUTS

mn_ReplyPort- pointer to message port that receives I/O request after

if the quick flag (IOF_QUICK) is clear

io_Device   - pointer to device node, must be set by (or copied from

I/O block set by) OpenDevice function

io_Unit     - bit map of channel to read (bit 0 thru 3 corresponds to

channel 0 thru 3), if more then one bit is set lowest

bit number channel read

io_Command  - command number for CMD_READ

io_Flags    - flags, must be cleared if not used:

IOF_QUICK - (CLEAR) reply I/O request

ioa_AllocKey- allocation key, must be set by (or copied from I/O block

set by) OpenDevice function or ADCMD_ALLOCATE command

OUTPUTS

io_Unit     - bit map of channel successfully read (bit 0 thru 3

corresponds to channel 0 thru 3)

io_Error    - error number:

0                    - no error

ADIOERR_NOALLOCATION - allocation key (ioa_AllocKey)

does not match key for channel

ioa_Data    - pointer to I/O block for current write, zero if none is

progress

audio.device/command/CMD_RESET                   audio.device/command/CMD_RESET

NAME

CMD_RESET -- restore device to a known state

FUNCTION

CMD_RESET is a standard command for multiple audio channels.  For each

selected channel (io_Unit), if the allocation key (ioa_AllocKey) is

correct, CMD_RESET:

. clears the hardware audio registers and attach bits,

. sets the audio interrupt vector,

. cancels all pending I/O (CMD_FLUSH), and

. un-stops the channel if it is stopped (CMD_STOP),

Otherwise, CMD_RESET returns an error (ADIOERR_NOALLOCATION).

CMD_RESET is synchronous and only replies (mn_ReplyPort) if the quick

flag (IOF_QUICK) is clear.  Do not use CMD_RESET in interrupt code at

interrupt level 5 or higher.

INPUTS

mn_ReplyPort- pointer to message port that receives I/O request

if the quick flag (IOF_QUICK) is clear

io_Device   - pointer to device node, must be set by (or copied from

I/O block set by) OpenDevice function

io_Unit     - bit map of channels to reset (bits 0 thru 3 correspond

to channels 0 thru 3)

io_Command  - command number for CMD_RESET

io_Flags    - flags, must be cleared if not used:

IOF_QUICK - (CLEAR) reply I/O request

ioa_AllocKey- allocation key, must be set by (or copied from I/O block

set by) OpenDevice function or ADCMD_ALLOCATE command

OUTPUTS

io_Unit     - bit map of channels to successfully reset (bits 0 thru 3

correspond to channels 0 thru 3)

io_Error    - error number:

0                    - no error

ADIOERR_NOALLOCATION - allocation key (ioa_AllocKey)

does not match key for channel

audio.device/command/CMD_START                   audio.device/command/CMD_START

NAME

CMD_START -- start device processing (like ^Q)

FUNCTION

CMD_START is a standard command for multiple audio channels.  For each

selected channel (io_Unit), if the allocation key (ioa_AllocKey) is

correct and the channel was previously stopped (CMD_STOP), CMP_START

immediately starts all writes (CMD_WRITE) to the channel.  If the

allocation key is incorrect, CMD_START returns an error

(ADIOERR_NOALLOCATION).  CMD_START starts multiple channels

simultaneously to minimize distortion if the channels are playing the

same waveform and their outputs are mixed. CMD_START is synchronous and

only replies (mn_ReplyPort) if the quick flag (IOF_QUICK) is clear.  Do

not use CMD_START in interrupt code at interrupt level 5 or higher.

INPUTS

mn_ReplyPort- pointer to message port that receives I/O request after

if the quick flag (IOF_QUICK) is clear

io_Device   - pointer to device node, must be set by (or copied from

I/O block set by) OpenDevice function

io_Unit     - bit map of channels to start (bits 0 thru 3 correspond

to channels 0 thru 3)

io_Command  - command number for CMD_START

io_Flags    - flags, must be cleared if not used:

IOF_QUICK - (CLEAR) reply I/O request

ioa_AllocKey- allocation key, must be set by (or copied from I/O block

set by) OpenDevice function or ADCMD_ALLOCATE command

OUTPUTS

io_Unit     - bit map of channels successfully started (bits 0 thru 3

correspond to channels 0 thru 3)

io_Error    - error number:

0                    - no error

ADIOERR_NOALLOCATION - allocation key (ioa_AllocKey)

does not match key for channel

audio.device/command/CMD_STOP                     audio.device/command/CMD_STOP

NAME

CMD_STOP -- stop device processing (like ^S)

FUNCTION

CMD_STOP is a standard command for multiple audio channels.  For each

selected channel (io_Unit), if the allocation key (ioa_AllocKey) is

correct, CMD_STOP immediately stops any writes (CMD_WRITE) in

progress; otherwise, CMD_STOP returns an error (ADIOERR_NOALLOCATION).

CMD_WRITE queues up writes to a stopped channel until CMD_START starts

the channel or CMD_RESET resets the channel. CMD_STOP is synchronous

and only replies (mn_ReplyPort) if the quick flag (IOF_QUICK) is

clear.  Do not use CMD_STOP in interrupt code at interrupt level 5 or

higher.

INPUTS

mn_ReplyPort- pointer to message port that receives I/O request after

if the quick flag (IOF_QUICK) is clear

io_Device   - pointer to device node, must be set by (or copied from

I/O block set by) OpenDevice function

io_Unit     - bit map of channels to stop (bits 0 thru 3 correspond to

channels 0 thru 3)

io_Command  - command number for CMD_STOP

io_Flags    - flags, must be cleared if not used:

IOF_QUICK - (CLEAR) reply I/O request

ioa_AllocKey- allocation key, must be set by (or copied from I/O block

set by) OpenDevice function or ADCMD_ALLOCATE command

OUTPUTS

io_Unit     - bit map of channels successfully stopped (bits 0 thru 3

correspond to channels 0 thru 3)

io_Error    - error number:

0                    - no error

ADIOERR_NOALLOCATION - allocation key (ioa_AllocKey)

does not match key for channel

audio.device/command/CMD_UPDATE                 audio.device/command/CMD_UPDATE

NAME

CMD_UPDATE -- force dirty buffers out

FUNCTION

CMD_UPDATE is a standard command for multiple audio channels.  For

each selected channel (io_Unit), if the allocation key (ioa_AllocKey)

is correct, CMD_UPDATE does nothing; otherwise, CMD_UPDATE returns an

error (ADIOERR_NOALLOCATION).  CMD_UPDATE is synchronous and only

replies (mn_ReplyPort) if the quick flag (IOF_QUICK) is clear.

INPUTS

mn_ReplyPort- pointer to message port that receives I/O request after

if the quick flag (IOF_QUICK) is clear

io_Device   - pointer to device node, must be set by (or copied from

I/O block set by) OpenDevice function

io_Unit     - bit map of channels to update (bits 0 thru 3 correspond

to channels 0 thru 3)

io_Command  - command number for CMD_UPDATE

io_Flags    - flags, must be cleared if not used:

IOF_QUICK - (CLEAR) reply I/O request

ioa_AllocKey- allocation key, must be set by (or copied from I/O block

set by) OpenDevice function or ADCMD_ALLOCATE command

OUTPUTS

io_Unit     - bit map of channels successfully updated (bits 0 thru 3

correspond to channels 0 thru 3)

io_Error    - error number:

0                    - no error

ADIOERR_NOALLOCATION - allocation key (ioa_AllocKey)

does not match key for channel

audio.device/command/CMD_WRITE                   audio.device/command/CMD_WRITE

NAME

CMD_WRITE -- normal I/O entry point

FUNCTION

CMD_WRITE is a standard command for a single audio channel (io_Unit).

If the allocation key (ioa_AllocKey) is correct, CMD_WRITE plays a

sound using the selected channel; otherwise, it returns an error

(ADIOERR_NOALLOCATION).  CMD_WRITE queues up requests if there is

another write in progress or if the channel is stopped (CMD_STOP).

When the write actually starts; if the ADIOF_PERVOL flag is set,

CMD_WRITE loads volume (ioa_Volume) and period (ioa_Period), and if

the ADIOF_WRITEMESSAGE flag is set, CMD_WRITE replies the write

message (ioa_WriteMsg).  CMD_WRITE returns an error (IOERR_ABORTED) if

it is canceled (AbortIO) or the channel is stolen (ADCMD_ALLOCATE).

CMD_WRITE is only asynchronous if there is no error, in which case it

clears the quick flag (IOF_QUICK) and replies the I/O request

(mn_ReplyPort) after it finishes writting; otherwise, it is synchronous

and only replies if the quick flag (IOF_QUICK) is clear.  Do not use

CMD_WRITE in interrupt code at interrupt level 5 or higher.

INPUTS

mn_ReplyPort- pointer to message port that receives I/O request after

the write completes

io_Device   - pointer to device node, must be set by (or copied from

I/O block set by) OpenDevice function

io_Unit     - bit map of channel to write (bit 0 thru 3 corresponds to

channel 0 thru 3), if more then one bit is set lowest

bit number channel is written

io_Command  - command number for CMD_WRITE

io_Flags    - flags, must be cleared if not used:

ADIOF_PERVOL       - (SET) load volume and period

ADIOF_WRITEMESSAGE - (SET) reply message at write start

ioa_AllocKey- allocation key, must be set by (or copied from I/O block

set by) OpenDevice function or ADCMD_ALLOCATE command

ioa_Data    - pointer to waveform array (signed bytes (-128 thru 127)

in custom chip addressable ram and word aligned)

ioa_Length  - length of the wave array in bytes (2 thru 131072, must

be even number)

ioa_Period  - sample period in 279.365 ns increments (124 thru 65536,

anti-aliasing filter works below 300 to 500 depending on

waveform), if enabled by ADIOF_PERVOL

ioa_Volume  - volume (0 thru 64, linear), if enabled by ADIOF_PERVOL

ioa_Cycles  - number of times to repeat array (0 thru 65535, 0 for

infinite)

ioa_WriteMsg- message replied at start of write,  if enabled by

ADIOF_WRITEMESSAGE

OUTPUTS

io_Unit     - bit map of channel successfully written (bit 0 thru 3

corresponds to channel 0 thru 3)

io_Flags    - IOF_QUICK flag cleared if there is no error

io_Error    - error number:

0                    - no error

IOERR_ABORTED        - canceled (AbortIO) or channel

stolen

ADIOERR_NOALLOCATION - allocation key (ioa_AllocKey)

does not match key for channel

BUGS

If CMD_WRITE starts the write immediately after stopping a previous

write, you must set the ADIOF_PERVOL flag or else the new data pointer

(ioa_Data) and length (ioa_Length) may not be loaded.

audio.device/Expunge                                       audio.device/Expunge

NAME

EXPUNGE - indicate a desire to remove the Audio device

FUNCTION

The Expunge routine is called when a user issues a RemDevice call.  By

the time it is called, the device has already been removed from the

device list, so no new opens will succeed.  The existence of any other

users of the device, as determined by the device open count being

non-zero, will cause the Expunge to be deferred.  When the device is

not in use, or no longer in use, the Expunge is actually performed.

audio.device/OpenDevice                                 audio.device/OpenDevice

NAME

OpenDevice - open the audio device

SYNOPSIS

error = OpenDevice("audio.device", unitNumber, iORequest, flags);

FUNCTION

The OpenDevice routine grants access to the audio device.  It takes an

I/O audio request block (iORequest) and if it can successfully open

the audio device, it loads the device pointer (io_Device) and the

allocation key (ioa_AllocKey); otherwise, it returns an error

(IOERR_OPENFAIL).  OpenDevice increments the open count keeping the

device from being expunged (Expunge).  If the length (ioa_Length) is

non-zero, OpenDevice tries to allocate (ADCMD_ALLOCATE) audio channels

from a array of channel combination options (ioa_Data). If the

allocation succeeds, the allocated channel combination is loaded into

the unit field (ioa_Unit); otherwise, OpenDevice returns an error

(ADIOERR_ALLOCFAILED).  OpenDevice does not wait for allocation to

succeed and closes (CloseDevice) the audio device if it fails.  To

allocate channels, OpenDevice also requires a properly initialized

reply port (mn_ReplyPort) with an allocated signal bit.

INPUTS

unitNumber- not used

iORequest - pointer to audio request block (struct IOAudio)

ln_Pri      - allocation precedence (-128 thru 127), only

necessary for allocation (non-zero length)

mn_ReplyPort- pointer to message port for allocation, only

necessary for allocation (non-zero length)

ioa_AllocKey- allocation key; zero to generate new key.

Otherwise, it must be set by (or copied from I/O

block that is set by) previous OpenDevice

function or ADCMD_ALLOCATE command (non-zero

length)

ioa_Data    - pointer to channel combination options (byte

array, bits 0 thru 3 correspond to channels 0

thru 3), only necessary for allocation (non-zero

length)

ioa_Length  - length of the channel combination option array

(0 thru 16), zero for no allocation

flags     - not used

OUTPUTS

iORequest - pointer to audio request block (struct IOAudio)

io_Device   - pointer to device node if OpenDevice succeeds,

otherwise -1

io_Unit     - bit map of successfully allocated channels (bits

0 thru 3 correspond to channels 0 thru 3)

io_Error    - error number:

0                   - no error

IOERR_OPENFAIL      - open failed

ADIOERR_ALLOCFAILED - allocation failed, no open

ioa_AllocKey- allocation key, set to a unique number if passed

a zero and OpenDevice succeeds

error     - copy of io_Error