ASPE (Amiga Serial Performance Evaluator) Release 1.2 (c)1990 CBM Inc.

=============================================dart========2/8/90=======

1.2 Enhancements:

-----------------

Everything is now variable. Well, almost ;->. There is now full command line

argument passing. Current arguments: which cia timer to use, baudrate, packet

size (number of bytes to send per interrupt) and total number of bytes per

test iteration. Type "aspe ?" or "aspe -h" for more info.

Abstract:

---------

ASPE sends data out the serial port at full bandwidth of a definable

baudrate and receives the same data. This requires connecting the receive

and transmit pins on the RS-232 port. You could hook up a MIDI interface and

connect MIDI IN to MIDI out, or connect pins 2 and 3 on a female DB25 jack.

You can test how activities, such as moving the mouse, selecting, moving and

sizing windows, accessing files etc, affect serial performance. You can use

different clock sources and packet sizes (good measure on the effect of

interrupt dispatch latencies). ASPE runs under 1.3 and 2.0 allowing you to

compare serial performance under different OS versions. ASPE may be helpful

in determining which functions in the system may need fine tuning to make

the OS more realtime.

Implementation:

---------------

The main task installs a cia timer interrupt, a serial receive interrupt

(both at software priority zero) and a stopwatch using the timer device. The

stopwatch is started before transmission is enabled. Serial transmission is

triggered by a cia timer interrupt which is signalling the main task. Upon

receiption of the signal the main task sends a definable number of bytes out

the RS-232 port. This saturates full serial bandwidth. Concurrently, a

serial receive interrupt handler is receiving bytes from the RS-232 port

keeping a count of the incoming bytes. A global counter variable,

incremented in the cia timer interrupt is counted up until it reaches a

definable number and stops the cia timer as well as the stopwatch.

The number of received bytes is subtracted from the number of sent bytes to

establish the number of dropped bytes. Statistics, including number of sent,

received and dropped bytes, as well as the total time elapsed, are printed

to the console. You are then returned to your shell.

Future:

-------

Variable densities and different transmit and receive algorithms will be

implemented soon. I might even add a full blown intuition interface 8-<

direct comments to: Darius Taghavy, CATS (x9425), e-mail darius@cbmvax