TABLE OF CONTENTS

misc.resource/MR_ALLOCMISCRESOURCE

misc.resource/MR_FREEMISCRESOURCE

misc.resource/MR_ALLOCMISCRESOURCE           misc.resource/MR_ALLOCMISCRESOURCE

NAME

MR_ALLOCMISCRESOURCE - allocate one of the misc resources

SYNOPSIS

CurrentUser = MR_ALLOCMISCRESOURCE( unitNum, name ), misc.resource

D0	                           D0       A1      A6

STRPTR CurrentUser;

LONG unitNum;

STRPTR name;

FUNCTION

This routine allocates one of the miscellaneous resources.

If the resource is currently allocated, an error is returned.

If you do get it, your name is associated with the resource

(so a user can see who has it allocated).

This routine may not be called from an interrupt routine

DESCRIPTION

There are certain parts of the hardware that a multitasking-

friendly program may need to take over.  The serial port

is a good example.  By grabbing the misc.resource for the

serial port, the caller would "own" the hardware registers

associated with that function.  Nobody else, including the

system serial driver is allowed to interfere.

The misc.resource must be accessed using assembly language.  The set

of currently defined units and the function offsets from the resource

base are listed in the resources/misc.i include file.

INPUTS

unitNum - the number of the resource you want to allocate

(eg.  MR_SERIALBITS).

name - a mnenonic name that will help the user figure out

what piece of software is hogging a resource.

(havoc breaks out if a name of null is passed in...)

RESULTS

CurrentUser - if the resource is busy, then the name of

the current user is returned.  If the resource is

free, then null is returned.

BUGS

SEE ALSO

resources/misc.i

misc.resource/MR_FREEMISCRESOURCE

misc.resource/MR_FREEMISCRESOURCE             misc.resource/MR_FREEMISCRESOURCE

NAME

MR_FREEMISCRESOURCE - make a resource available for reallocation

SYNOPSIS

MR_FREEMISCRESOURCE( unitNum ), misc.resource

D0         A6

LONG unitNum;

FUNCTION

This routine frees one of the resources allocated

by MR_ALLOCMISCRESOURCE.  The resource is made available

for reuse.

This routine may not be called from an interrupt routine.

INPUTS

unitNum - the number of the miscellaneous resource to be freed.

RESULTS

Frees the appropriate resource.

BUGS

SEE ALSO

misc.resource/MR_ALLOCMISCRESOURCE