Introduction to 1.3 IEEE Double Precision Libraries

===================================================

by Dale Luck

Beta 2 Release

The basic double precision IEEE library has been rewritten for 1.3

The new library is up to seven times faster than the old one and

averages twice as fast for +,-,*,/.  It more properly handles

IEEE special cases like NotANumber, Infinity, and denormalized numbers.

There is a table of approximate speed-ups per function at the end

of this article.

Besides the faster software emulation of floating point, this

library recognizes and uses the 68020/68881* combination and

will use the floating point instructions available. Also, if an

preexisting math resource** is available it will use that as well.

Normally this resource will point to the base of a 68881 designed

as a 16 bit i/o port, but maybe a different device instead.

We also provide the ability to programmatically trap math errors such

as overflow and divide by zero. Your program can now ignore them

or take suitable action without visiting the GURU.

A brand new library has been written for 1.3. It is the IEEE

double precision transcendental library. It supports the same functions

as the transcendental library for the Motorola fast floating point.

This library also identifies and uses the 68020/68881 combination as

well as the math resource. It has a very fast software square root routine.

Why should you use these libraries?

These libraries are highly optimized and provide very fast emulation

of IEEE double functions.

Your code can share the IEEE libraries with other applications.

This saves 16k bytes per user with the transcendental library.

If you need the precision of IEEE double, and wish to have a transparent

improvement in speed when your programs run on machines with math

coprocessors, then you should use these libraries. All the decision

making is done by the library when it is first initialized and it will

use the fastest available resources to do your math. You only need

one program to support a standard Amiga, a 68020/68881 Amiga, or an

external math coprocessor Amiga.

Why wouldn't you use these libraries?

For ultimate speed on a 68020/68881 system, you will want to use

the new instructions available from the 68020/68881 directly in

your code. This, however, prevents your code from running on conventional

68000 powered Amigas. In this case, you may need to supply different

versions of your code, one for each different configuration.

These libraries should be placed in the libs: directory.

The basic library will replace the old library, and the transcendental

library is an addition.

For programmers there are additional files to link with as well as

a new .fd file for the transcendental functions.

* There are several manufactures of 68020/68881 upgrade boards such

as CSA. Commodore-Amiga has also announced such a product.

** Math Resources have been announced by Microbotics and ASDG.

Hardware Developer information.

To make use of CBM's standard peripheral support for 68881 you

must design your peripheral to autoconfig. Your autoconfig

software must create a resource and add it to the resource list.

The name of this resource is "MathIEEE.resource".

The IEEE library will attempt to open this resource. If it finds it,

it will extract the BaseAddr pointer and copy it into its library

structure. If the BaseAddr pointer is non-null it will use a different

list of routine entry points when the IEEE library is initialized.

After the IEEE library is initialized, the library again checks the

resource for alternate function bits in Flags of the resource. The Basic

library only checks the

DblBasAlt bit, and the transcendental library only checks the

DblTransAlt bit. If they are set, the library routine will call the

function whose address is in the corresponding Init field.

The arguments passed are a6 = sysbase,a1=resource,a2=mathlibrary.

If your device is not a 68881 then you may need to use this.

There are separate bits for different library capabilities in case

your math resource is only able to handle a limited set of functions.

This will let you tie a math processor in that may only provide

add/subtract/multiply/divide. The rest of software  will

use it transparently by calling your alternate routines.

Amiga does not provide for arbitrating a math accelerator in

a multitasking environment. So you must provide some support there

when your device autoconfigs. The only exception is the 68020/68881

combination where support for that has been standard since V1.2

Arbitration usually involves saving and restoring the state of you

hardware device between task switches.

We recommend that you look at the tc_Switch/tc_Launch vectors in

the task data structure. These are called each time control transfers

from one task to another. And remember not to assume that you are

the only process needing to use those vectors.

The resource data structure is as follows:

STRUCTURE  MathIEEE,LN_SIZE

UWORD   MathIEEE_Flags

ULONG   MathIEEE_BaseAddr       ; for standard 68881 support

ULONG   MathIEEE_DblBasInit     ; something else besides 68881

ULONG   MathIEEE_DblTransInit   ; something else besides 68881

ULONG   MathIEEE_SnglBasInit    ; something else besides 68881

ULONG   MathIEEE_SnglTransInit  ; something else besides 68881

LABEL  MathIEEE_sizeof

* the MathIEEE resource structure may grow in the future. Extensions

* will be added only as Amiga may define new capabilities. Such

* as 80 bit extended format.

*       bits for MathIEEE_flags

*       all unassigned bits must be 0

BITDEF  MathIEEE,DblBasAlt,0            ; alternate Basic library

BITDEF  MathIEEE,DblTransAlt,1          ; alternate Trans library

BITDEF  MathIEEE,SnglBasAlt,2           ; alternate Basic library

BITDEF  MathIEEE,SnglTransAlt,3         ; alternate Trans library

*       The 'Init' entries are only used if the corresponding

*       Bit is set in the Flags field

*       So if you are just a 68881, you do not need the Init stuff

*       just make sure you have cleared the Flags field

*       This should allow us to add Extended Precision later

*       For Init users, make sure you splice yourself into the

*       Open/Close/Expunge vectors for this library

The library structure that is used is tentatively laid out this way.

I say tentatively because the name of the entries may change yet, but

I will not change their order, usage, or size.

Naturally we  may add stuff to the end.

STRUCTURE  MI,LIB_SIZE      ; Standard library node

UBYTE   io8_Flags       ; is this 68881?

UBYTE   io8_pad         ; line up to next 32bit boundary

ULONG   io8_68881       ; ptr to io68881 base

ULONG   io8_SysLib      ; ptr to SysBase

ULONG   io8_SegList     ; ptr to this SegList

ULONG   io8_Resource    ; ptr to mathIEEE.resource

ULONG   io8_opentask    ; called when task opens

ULONG   io8_closetask   ; called when task closes

LABEL   MI_SIZE

Of particular interest to hardware developers are the opentask/closetask

entry points. These functions will be called when a task calls

OpenLibrary and CloseLibrary. This will give the vendor the opportunity

to set up any per task initialization necessary. The Amiga library

presently sets them up as NOPs in the case of straight emulation.

It puts the 68881 initialization stuff in there for 68020/68881 as well

as the peripheral 68881. That initialization stuff currently sets up

rounding modes and interrupt requests.

If you need to override the defaults, you will have to set the appropriate

Alt bits in the Resource structure and overwrite the opentask/closetask

fields when your AltInit function is called. The OpenLibrary routine

checks the return value of opentask for errors. If a nonzero is in d0.l

then OpenLibrary will return 0 to the task trying to OpenLibrary.

Table of Approximate Speed Ups from 1.2 to 1.3

--------------------------------------------------

Function          Times faster in 1.3/68000

==================================================

IEEEDPFlt		2.1

IEEEDPFix		5.0

IEEEDPAbs		1.8*

IEEEDPNeg		3.0*

IEEEDPTst		2.8

IEEEDPFloor		7.6

IEEEDPCeil		7.2

IEEEDPCmp		2.8*

IEEEDPAdd		1.9

IEEEDPSub		1.8

IEEEDPMul		2.1

IEEEDPDiv		2.1

* updated for Gamma1 release of ieee library.

Test program was written in C and used the standard IEEEDP interface

routines. Results for assembly language programmers will probably be

better if you use inline jsr _LVOxxx(a6)'s.

A 68010 improved results about 5% for multiply and about 9% for divide.

On the 68020/68881 some new exceptions are generated. Unfortunately

the V1.2/V1.3 kickstart roms do not properly initialize these. This

has been fixed for a future release of the operating system. In 1.3

workbench disk we have added a program to fix exec as well as fix

some other os bugs. This program is called setalert, and it should

be put in your startup-sequence. See the 1.3 documentation for more

information.

For those that have not gotten 1.3 yet, the setalert program is

included on this disk. You should use 'run setalert' since this

program does not return but hanges around to do it's work.