Copyright (c) 1986, Commodore-Amiga Inc.  All rights reserved.

this document:	drivers.doc

last changed:	26 March 1986, Neil Katin

This document talks about how the "binddrivers" program finds your

driver and links it into the system.  It also hints on how

you should write your code to take advantage of this.

This code is very preliminary.  If you find any bugs, or have

suggestions for the documentation, please let me know!  I will

try and maintain compatibility, but may change things to correct

design flaws or add useful enhancements.  Rember that this is

still in alpha testing.

First off, the expansion library goes out and configures the expansion

boards in the system.  It puts each board in its own address space,

and links memory boards into the memory free pool.  This is done

by the expansion.library's ConfigChain entry point.  This code

is intended to be run early on in system startup, before any

other code is around.

Later on, after the dos is running, the binddrivers program should

be run.  This program searches the directory "SYS:Expansion" for

workbench icon files.  If it finds one with a tooltypes variable

"PRODUCT" then it parses the rest of the line (see below) and

looks for an unconfigured board that matches the description.

This method makes user installation of a new driver trivial:

the user only has to copy a workbench icon into the expansion

directory on his sys disk.  Everything else is automatic the

next time he boots.

In addition, the binddrivers program may be run repeatedly without

ill effect.  Devices will not be configured twice, so binddrivers

may be run after a new driver is installed (so the user does not

have to reboot after installing a driver).

Here is an overview of the process:

search:

for each file that ends in .info, do test()

test:

1.  Call GetDiskObject() on this file.  If not a workbench object,

return.

2.  Call FindToolType() to see if there is a PRODUCT definition.

If not, return.

3.  If the description does not match an unconfigured board, return.

If there are boards, link them all together and record them

in a static area.

4.  LoadSeg() the code file.  If LoadSeg fails, return.

5.  Search the first hunk for a Resident structure.  If no structure,

UnLoadSeg() the segment and return.

6.  InitResident() the loaded code.  If an error (NULL) is returned,

UnLoadSeg() the segment.

your driver code:

Find the list of boards.  Mark them a configured, and record

your driver in them (for system debugging).  return non-zero

value if everything went ok.  If something went wrong (or you

just want to be unloaded) then return NULL.

Now for some more detail.

1.  GetDiskObject() is a routine in icon.library.  It will read in

the disk object, and return a pointer to it.  Part of a disk object

structure is a "tooltypes" field.

2.  The FindToolType() routine (also in the icon.library) searches

the tooltypes database associated with the disk object.  If there

is an entry for PRODUCT then it is assumed that this is an info

file for a driver.  The PRODUCT field is of the format:

PRODUCT=<idlist>

<idlist> ::= <id> | <idlist>BAR<id>

<id> ::= <manufacturer> | <manufacturer>SLASH<product>

<manufacturer> ::= <a decimal number>

<product> ::= <a decimal number>

BAR ::= <a vertical bar -- '|'>

SLASH ::= <a forwards slant char -- '/'>

Spaces are not legal.  Some examples:

PRODUCT=1000/30			; matches man 1000, product 30

PRODUCT=1000			; matches any man 1000 board

PRODUCT=1000/20|1000/21		; matches man 1000, product 20 or 21

3.  Each unconfigured board in the system is searched.  An unconfigured

board has the CDB_CONFIGME bit set in the cd_Flags byte.  Search

all these unconfigured boards to find the ones that matche any of the

product codes.  Link all these boards together using the cd_NextCD

field of the ConfigDev structure.  Record the head of this list,

along with the product field and the name of the file that was

loaded in a CurrentBinding structure.  This structure may be

retrieved via the GetCurrentBinding() call.

4.  Attempt to load in the driver.  The driver may be a device, library,

task, process, or anything else that you may want.  The only requirement

is that it have a Resident structure in its first hunk.  (By the

way, this means that you can not directly use startup.obj).  This

is why we refer to loading a "driver" rather than a "device" -- you

can write any sort of code you want to handle your device.

5.  Binddriver will search the first hunk for a Resident structure.

If it cannot find one, it will assume some awful mistake has

been made, and will unload the segment.

6.  Finally we get to running some of YOUR code.  InitResident() is

used to start you off and running.  The return value from

InitResident (and therefore the return value from your init

entry point) will be checked on exit.  If it is zero then the

segment will be unloaded.  This can be useful if you only

need to do a bit of initialization and then can go away, such

as allocate additional expansion memory for a non-expansion

architecture board.

Hints for writing your driver code:

Your driver will be launched via InitResident() as discussed

above.  If you use the underdocumented, but very useful

RT_AUTOINIT option you will have a library node constructed

for you, and then have the code you specified entered.  If you

don't use AUTOINIT, then your code will be entered directly.

You should (among everything else you might be doing) open

the expansion.library and ask for the current bindings

(GetCurrentBinding()).  In this structure will be the head

of a singly linked list of ConfigDev structures.  The structures

are linked via the cd_NextCD field.  You should deal with

each member of the list -- they are for you!

There are two actions that you MUST take.  One is to unset

the CDB_CONFIGME bit in the cd_Flags.  If you do not do this

then the board is still available for other drivers (of course,

you may actually want this...).  If you do unset the CONFIGME

bit, please also record your "node" in the cd_Driver structure.

It is assumed that this is an exec node, whose LN_NAME field

points your your name, and LN_TYPE field is your type of "thing" --

libarary, resource, device, task, etc.  I know that this will

not always hold, but try it anyway.  It will help the rest of

us debug the system when something goes wrong.

You have now done everything you wanted to.  Your init routine

is about to return.  If you return a zero, then your code

will be unloaded.  If you return non-zero then you will stay

around.

Well, thats it.  Hope it helps.