TalkCD will allow you to talk directly to the audio drive

mechanism.  To send a command, simply type the bytes to be

sent in the "Send_To_Drive" window and press return.  The

index of the packet (first four bits) are calculated

automatically.  The checksum is also calculated

automatically when a "*" is entered.  The "*" is required

because that is how the program determines where the end of

the packet is.  Examples:

7 *                         (send ID packet)

4 0 2 0 1 20 0 0 0 0 4 0 *  (play from 00:02:00 to 01:20:00)

5 1 *                       (Turn on drive light)

5                 (same thing -- the * is the end of the

1                                packet, not the RETURN)

*

Warning:  If you turn on automatic Q-Code reporting while

doing a SETPLAY, the "Receive_From_Drive" window will

probably not be able to print the packets fast enough and

the buffer will wrap and the program will get out of sync.

When this happens, the data you get back in the receive

buffer will not match the actual data the drive is sending

back.

Solution:  Don't do it.

Note to Philips:

The following command should send back a response:

05 81 *

the high bit of the second byte indicates that a response to

the packet is requested.  The drive does not respond.