TABLE OF CONTENTS

trackdisk.device/TD_ADDCHANGEINT

trackdisk.device/TD_CHANGENUM

trackdisk.device/TD_CHANGESTATE

trackdisk.device/TD_FORMAT

trackdisk.device/TD_GETDRIVETYPE

trackdisk.device/TD_GETNUMTRACKS

trackdisk.device/TD_MOTOR

trackdisk.device/TD_PROTSTATUS

trackdisk.device/TD_RAWREAD

trackdisk.device/TD_RAWWRITE

trackdisk.device/TD_REMCHANGEINT

trackdisk.device/TD_SEEK

trackdisk.device/TD_ADDCHANGEINT               trackdisk.device/TD_ADDCHANGEINT

NAME

TD_ADDCHANGEINT - add a new change software int

SYNOPSIS

TDUAddChangeInt( IORequest ), UnitPtr

A1           A3

FUNCTION

Alas, the old TDURemove call was not robust enough.  This routine

supports an extensible list of software interrupts for use by many

different supporting drivers.

The call does not "complete" (e.g. TermIO).  The request

is stashed until TDURemChangeInt is called, when it is

finally replied.

INPUTS

IORequest - a standard IO Request block (IO_DATA-> soft int struct).

RESULTS

EXCEPTIONS

SEE ALSO

BUGS

trackdisk.device/TD_CHANGENUM                     trackdisk.device/TD_CHANGENUM

NAME

TD_CHANGENUM - return the current disc change number

SYNOPSIS

TDUChangeNum( IORequest ), UnitPtr

A1           A3

FUNCTION

This routine checks to see if there is a disc in the drive

of the specified unit.

INPUTS

IORequest - a standard IO Request block

RESULTS

EXCEPTIONS

SEE ALSO

BUGS

trackdisk.device/TD_CHANGESTATE                 trackdisk.device/TD_CHANGESTATE

NAME

TD_CHANGESTATE - Return the current state of the disc

SYNOPSIS

TDUChangeState( IORequest ), UnitPtr

A1           A3

FUNCTION

This routine checks to see if there is a disc in the drive

one the specified unit.

INPUTS

IORequest - a standard IO Request block

RESULTS

IO_ACTUAL -- nonzero if there is no diskette in the drive

EXCEPTIONS

SEE ALSO

BUGS

trackdisk.device/TD_FORMAT                           trackdisk.device/TD_FORMAT

NAME

TD_FORMAT -- format the entire disc

SYNOPSIS

TDUFormat( iOBlock ), DevNode

D0         A1         A6

FUNCTION

The function formats the entire disc, destroying all data.

It fills all the sectors with the contents of the iOBlock.

The iOBlock must point to (at least) one sector worth of

information.  Any info greater than one sector is ignored.

NO ERROR CHECKING is done

INPUTS

RESULTS

SEE ALSO

trackdisk.device/TD_GETDRIVETYPE               trackdisk.device/TD_GETDRIVETYPE

NAME

TD_GETDRIVETYPE - return the type of the disk drive to the user

FUNCTION

This routine returns the type of the disk to the user.

This number will be a small integer.  It will come from

the set of DRIVE... defines in trackdisk.h

or trackdisk.i.

The only way you can get to this call is if the trackdisk

device understands the drive type of the hardware that is

plugged in.  This is because the OpenDevice call will fail

if the trackdisk device does not understand the drive type.

To find raw drive identifiers see the disk resource's

DR_GETUNITID entry point.

IO REQUEST

io_Command      TD_GETDRIVETYPE

RESULTS

io_Actual       the drive type connected to this unit.

SEE ALSO

TD_GETNUMTRACKS

trackdisk.device/TD_GETNUMTRACKS               trackdisk.device/TD_GETNUMTRACKS

NAME

TD_GETNUMTRACKS - return the number of tracks on this type of disk

FUNCTION

This routine returns the number of tracks that are available

on this disk unit.  This call obsoletes the older NUMTRACKS

hard coded constant.

IO REQUEST

io_Command      TD_GETNUMTRACKS

RESULTS

io_Actual       number of tracks accessible on this unit

SEE ALSO

TD_GETDRIVETYPE

trackdisk.device/TD_MOTOR                             trackdisk.device/TD_MOTOR

NAME

TD_MOTOR - user visible control for motor

SYNOPSIS

TDUMotor( IOBlock ), UnitPtr, DevPtr

A1         A3       A6

FUNCTION

This routine allows the user to control the disc motor. He

may turn it either on or off.  Note that the motor will be

automatically turned on during an I/O request, but is never

turned of except by this command.

INPUTS

IOBlock - the command block for this IO operation.

IO_ACTUAL -- returns the previous state of the motor

IO_LENGTH -- the requested state of the motor

0 ==> turn motor off

1 ==> turn motor on

EXCEPTIONS

SEE ALSO

BUGS

trackdisk.device/TD_PROTSTATUS                   trackdisk.device/TD_PROTSTATUS

NAME

TD_PROTSTATUS -- return whether the current disk is write protected

SYNOPSIS

TDUProtstatus( IOBlock ), UnitPtr, DevPtr

A1         A3       A6

FUNCTION

This routine tells whether the current disk is write protected.

INPUTS

IOBlock - the command block for this IO operation.

IO_ACTUAL - nonzero if the disk is protected, 0 otherwise

If there is no disk in the drive, then IO_ERROR is set

to TDERR_DiskChanged

EXCEPTIONS

SEE ALSO

BUGS

trackdisk.device/TD_RAWREAD                         trackdisk.device/TD_RAWREAD

NAME

TD_RAWREAD - read a raw sector from the disk

FUNCTION

This routine performs a raw read for the track disk.

It seeks to the specified track and reads it in to the

user's buffer.  This buffer MUST be in chip memory.

NO PROCESSING OF THE TRACK IS DONE.  It will appear exactly

as the bits come out off the disk -- hopefully in some legal MFM

format (if you don't know what MFM is, you shouldn't be using

this call...).  Caveat Programmer.

This interface is intended for sophisticated programmers

only.  Commodore-Amiga may make enhancements to the disk

format in the future.  We will provide compatibility

within the trackdisk device.  Anyone who uses this routine

is bypassing this upwards compatibility.  If your application

breaks, TOUGH!

If this warning is not enough, then add suitable additional

harrassment of your choice.

IO REQUEST

io_Flags        if the IOTDB_INDEXSYNC bit is set then the driver

will make a best effort attempt to start reading

from the index mark.  Note that there

will be at least some delay, and perhaps a great

deal, of delay (if, for example, interrupts have

been Disabled()...).

io_Command      TD_RAWREAD or ETD_RAWREAD.

io_Length       Length of buffer (in bytes).  The maximum allowable

length is 32K bytes.

io_Data         Pointer to buffer in chip memory where raw track

will be read into.

io_Offset       The track number to read in (not this is different

from a normal trackdisk io call which is given

in terms of logical bytes from the beginning of

the disk.  This is because the trackdisk driver

has no idea what the format of the disk is).

iotd_Count      (ETD_RAWREAD only) maximum allowable change counter

value

RESULTS

io_Error        non-zero if there was an error

LIMITATIONS for synced reads and writes

There is a delay between the index pulse and the start of bits

coming in from the drive (e.g. dma started).  This delay

is in the range of 135-200 micro seconds.  This delay breaks

down as follows: 55 microsecs is software interrupt overhead

(this is the time from interrupt to the write of the DSKLEN

register).  66 microsecs is one horizontal line delay (remember

that disk io is synchronized with agnus' display fetches).

The last variable (0-65 microsecs) is an additional scan line

since DSKLEN is poked anywhere in the horizontal line.  This leaves

15 microsecs unaccounted for...  Sigh.

In short, You will almost never get bits withing the first 135

microseconds of the index pulse, and may not get it until 200

microseconds.  At 4 microsecs/bit, this works out to be between

4 and 7 bytes of user data of delay.

SEE ALSO

TD_RAWWRITE

trackdisk.device/TD_RAWWRITE                       trackdisk.device/TD_RAWWRITE

NAME

TD_RAWWRITE - write a raw sector to the disk

FUNCTION

NO PROCESSING OF THE TRACK IS DONE.  The disk will appear exactly

as the bits come out of memory -- hopefully in some legal MFM

format (if you don't know what MFM is, you shouldn't be using

this call...).  Caveat Programmer.

NO PROCESSING OF THE TRACK IS DONE.  It will exactly

as the bits come out off the disk.  Caveat Programmer.

This interface is intended for sophisticated programmers

only.  Commodore-Amiga may make enhancements to the disk

format in the future.  We will provide compatibility

within the trackdisk device.  Anyone who uses this routine

is bypassing this upwards compatibility.  If your application

breaks, TOUGH!

If this warning is not enough, then add suitable additional

harrassment of your choice.

IO REQUEST

io_Flags        if the IOTDB_INDEXSYNC bit is set then the driver

will make a best effort attempt to start writing

from the index mark.  Note that there

will be at least some delay, and perhaps a great

deal, of delay (if, for example, interrupts have

been Disabled()...).

io_Command      TD_RAWWRITE or ETD_RAWWRITE.

io_Length       Length of buffer (in bytes).  The maximum allowable

length is 32K bytes.

io_Data         Pointer to buffer in chip memory where raw track

will be read into.

io_Offset       The track number to read in (not this is different

from a normal trackdisk io call which is given

in terms of logical bytes from the beginning of

the disk.  This is because the trackdisk driver

has no idea what the format of the disk is).

iotd_Count      (ETD_RAWWRITE only) maximum allowable change counter

value

RESULTS

io_Error        non-zero if there was an error

LIMITATIONS for synced reads and writes

There is a delay between the index pulse and the start of bits

going out to the drive (e.g. write gate enabled).  This delay

is in the range of 135-200 micro seconds.  This delay breaks

down as follows: 55 microsecs is software interrupt overhead

(this is the time from interrupt to the write of the DSKLEN

register).  66 microsecs is one horizontal line delay (remember

that disk io is synchronized with agnus' display fetches).

The last variable (0-65 microsecs) is an additional scan line

since DSKLEN is poked anywhere in the horizontal line.  This leaves

15 microsecs unaccounted for...  Sigh.

In short, You will almost never get bits withing the first 135

microseconds of the index pulse, and may not get it until 200

microseconds.  At 4 microsecs/bit, this works out to be between

4 and 7 bytes of user data of delay.

SEE ALSO

TD_RAWREAD

trackdisk.device/TD_REMCHANGEINT               trackdisk.device/TD_REMCHANGEINT

NAME

TD_REMCHANGEINT - remove a change software int

SYNOPSIS

TDURemChangeInt( IORequest ), UnitPtr

A1           A3

FUNCTION

This function unlinks the IOReqest stashed by AddChangeInt.

It also replies it to the user.

INPUTS

IORequest - a standard IO Request block

RESULTS

EXCEPTIONS

SEE ALSO

BUGS

trackdisk.device/TD_SEEK                               trackdisk.device/TD_SEEK

NAME

TD_SEEK - user visible control for the heads

SYNOPSIS

TDUSeek( IOBlock ), TDLib

A1         A6

FUNCTION

This routine allows the user to control the seek position.

Note that the heads will be automatically seeked during an

I/O request; this command allows the heads to be preseeked

if the next position is known prior to the I/O being ready.

INPUTS

IOBlock - the command block for this IO operation.

IO_OFFSET -- the location to seek to

EXCEPTIONS

SEE ALSO

BUGS